# UNIT I – INTRODUCTION TO COMPUTER NETWORKS AND PHYSICAL LAYER

## 1. Introduction to Network Models

### 1.1 OSI Model (Open Systems Interconnection)

- OSI Model is a conceptual framework used to understand and standardize network communication.

- 7 Layers explained in detail:

| Layer | Function | Example |
| --- | --- | --- |
| Physical | Transfers raw bits over a medium. Deals with voltage levels, cables, and connectors. | Ethernet cables, USB, Fiber optics |
| Data Link | Provides reliable transfer of data frames. Handles error detection & correction. | Switches, MAC addresses |
| Network | Handles logical addressing and routing of packets. | IP addressing, Routers |
| Transport | Ensures end-to-end delivery, flow control, and error recovery. | TCP, UDP |
| Session | Manages sessions and dialogues between applications. | Remote login (Telnet), NetBIOS |
| Presentation | Translates data into a standardized format, encryption/decryption, compression. | JPEG, ASCII, MPEG |
| Application | Interface for user applications to access network services. | HTTP, FTP, SMTP |

Example of OSI in real-world:

- Sending an email: Application Layer → Presentation Layer (formats) → Session Layer (opens connection) → Transport Layer (TCP) → Network Layer (IP addressing) → Data Link Layer (MAC address) → Physical Layer (bits sent over wire).

### 1.2 TCP/IP Model

- 4 layers of TCP/IP model:

| Layer | Function | Example |
| --- | --- | --- |
| Network Access | Handles physical transmission and MAC addressing. | Ethernet, Wi-Fi |
| Internet | Responsible for logical addressing and routing of data. | IP addressing, ICMP |
| Transport | End-to-end delivery and reliability. | TCP (reliable), UDP (unreliable) |
| Application | Provides network services to applications. | HTTP, FTP, SMTP, DNS |

- Key Differences OSI vs TCP/IP:

  - OSI is theoretical, TCP/IP is practical and widely used

  - OSI has 7 layers, TCP/IP has 4 layers

  - OSI separates presentation and session layers; TCP/IP combines them in the application layer

Example:

- Accessing a website uses HTTP over TCP/IP, which maps onto TCP/IP layers rather than OSI exactly.

### 1.3 Examples of Networks

- Novell Networks: Early LAN networks using NetWare operating system

- ARPANET: First packet-switching network, the foundation of the modern Internet

- Internet: Global system connecting millions of private, public, academic, and business networks

- Enterprise Networks: Office or campus networks connecting computers, servers, and devices

- Home Networks: LAN connecting PCs, phones, smart devices

Example Scenario:

- An office network uses a LAN for file sharing, connected to WAN for cloud access.

### 1.4 Network Topologies

- Topology = Layout of network devices (logical & physical).

| Topology | Description | Example | Advantages | Disadvantages |
| --- | --- | --- | --- | --- |
| Bus | All devices share one communication line | Small office LAN | Easy to install | Collision, difficult troubleshooting |
| Star | Devices connected to central hub/switch | Office network | Easy to manage, isolate faults | Hub failure stops network |
| Ring | Devices connected in a circle | Token Ring LAN | No collisions | One node failure disrupts network |
| Mesh | Multiple paths between devices | WAN backbone | Fault-tolerant | Expensive, complex wiring |
| Tree | Hierarchical combination of star & bus | Campus network | Scalable | Complex |

Example:

- School LAN: Star topology for each building, connected to central server forming tree topology.

## 2. Physical Layer

- Physical layer defines hardware specifications and transmission mechanisms for sending raw bits over the medium.

### 2.1 Transmission Media

- Copper cables:

  - Twisted pair: UTP, STP for Ethernet networks

  - Coaxial: High bandwidth for cable TV and broadband

- Fiber optics:

  - Uses light pulses to transmit data

  - Advantages: Very high speed, long distance, low interference

- Wireless Media:

  - Wi-Fi, Bluetooth, Satellite, Microwave

  - Suitable for mobility and hard-to-wire environments

Example:

- Fiber optics: Backbone of Internet providing 1 Gbps+ speeds

- Wi-Fi: Home wireless network connecting devices without cables

### 2.2 Switching Techniques

- Circuit Switching:

  - Dedicated path established for the duration of the connection

  - Example: Telephone networks

- Packet Switching:

  - Data divided into packets, sent independently

  - Routers forward packets toward destination

  - Example: Internet, LANs

- Message Switching (optional):

  - Entire message stored and forwarded by intermediate nodes

  - Used in email systems

### 2.3 Encoding and Asynchronous Communication

- Encoding: Converts data into signals for transmission

  - Examples: NRZ (Non-Return to Zero), Manchester, Bipolar

- Asynchronous Communication:

  - Data sent byte-by-byte with start and stop bits

  - Used in low-speed serial ports

Example:

- Keyboard inputs ASCII characters asynchronously to CPU

### 2.4 Bandwidth and Services

- Narrowband: Low-frequency, single signal (voice)

- Broadband: High-frequency, multiple simultaneous signals (Internet)

- ISDN: Digital transmission over standard telephone lines, multiple channels

- ATM (Asynchronous Transfer Mode): Cell-based switching for voice, video, and data

Example:

- Video conferencing uses broadband or ATM for high-speed data transfer

### 3. Summary

- Network Models: OSI (theoretical), TCP/IP (practical)

- Network Types: LAN, WAN, MAN

- Topologies: Star, Bus, Ring, Mesh, Tree

- Physical Layer: Transmission media, encoding, switching, communication methods

- High-speed services: ISDN, ATM, Broadband

### 4. Practice Problems

- Compare OSI and TCP/IP models and give examples of protocols for each layer.

- Describe advantages and disadvantages of star and bus topologies with examples.

- Explain circuit switching vs packet switching.

- What are ISDN and ATM, and how do they differ?

- List types of transmission media and give applications for each.

# UNIT II – DATA LINK LAYER & MEDIUM ACCESS CONTROL

## 1. Data Link Layer – Overview

- Purpose: Ensures reliable node-to-node data transfer over a physical link.

- Position: Layer 2 in OSI model, between Physical layer and Network layer.

Functions:

- Framing

- Error detection and correction

- Flow control

- Medium access control (if shared medium)

## 2. Design Issues of Data Link Layer

- Framing:

  - Divides a stream of bits from physical layer into frames for reliable transmission.

  - Techniques:

    - Character Count – Each frame starts with count of characters.

      - Problem: Count can be corrupted → framing error

    - Flag Bytes with Byte Stuffing – Special start/end flags (01111110). Extra ‘0’ inserted if pattern occurs in data.

    - Bit Stuffing – Adds a 0 after 5 consecutive 1s in data.

Example: In HDLC, bit stuffing is used to avoid confusion with flag sequence.

## 3. Error Detection and Correction

- Errors: Caused by noise, interference, or signal attenuation.

### 3.1 Types of Errors

- Single-bit error

- Burst error (multiple bits in a sequence)

### 3.2 Techniques for Error Detection

- Parity Check: Adds a parity bit (even or odd).

  - Example: 1011001 → Add parity bit 0 (even parity).

- Checksum: Sum of data bytes transmitted; receiver checks sum.

- Cyclic Redundancy Check (CRC): Uses polynomial division. Very effective for burst errors.

### 3.3 Error Correction

- Automatic Repeat Request (ARQ): Retransmit frame if error detected

  - Example: Stop-and-Wait ARQ, Go-Back-N, Selective Repeat

- Forward Error Correction (FEC): Add redundant bits so receiver can correct errors without retransmission

## 4. Elementary Protocols

### 4.1 Stop-and-Wait Protocol

- Transmitter sends one frame and waits for ACK before sending next frame.

- Advantage: Simple

- Disadvantage: Inefficient for long-distance links

Example:

- Transmission delay = 10 ms, frame size = 1000 bits → channel idle while waiting for ACK

### 4.2 Sliding Window Protocol

- Multiple frames can be sent before receiving ACK

- Window size = number of frames allowed to be unacknowledged

- Types: Go-Back-N, Selective Repeat

Example:

- Window size = 4 → transmitter sends 4 frames before needing ACK

### 4.3 SLIP (Serial Line Internet Protocol)

- Simple protocol for framing packets over serial links

- Ends each packet with END character (0xC0)

## 5. Data Link Layer in Common Networks

- HDLC (High-level Data Link Control):

  - Bit-oriented protocol for point-to-point and multipoint links

  - Uses framing, bit-stuffing, CRC, and sliding window

- Internet:

  - Ethernet at Data Link Layer uses MAC addressing and CRC for error detection

- ATM (Asynchronous Transfer Mode):

  - Cell-based network, fixed-size cells (53 bytes), uses CRC for error detection

## 6. Medium Access Control (MAC) Sub-layer

- Purpose: Controls access to shared medium in LANs/WLANs

### 6.1 MAC Protocols

- ALOHA: Random access protocol

  - Send whenever ready; collisions are retransmitted

  - Efficiency: ~18% for pure ALOHA

- Carrier Sense Multiple Access (CSMA):

  - Sense channel before transmitting

  - Variants:

    - CSMA/CD (Collision Detection) → Ethernet

    - CSMA/CA (Collision Avoidance) → Wi-Fi

- MAC Addresses:

  - Unique 48-bit hardware address for devices in LAN

  - Used for frame delivery at Data Link Layer

## 7. LAN and Wireless Standards

- IEEE 802.x Standards:

  - 802.3 (Ethernet): Wired LAN

  - 802.11 (Wi-Fi): Wireless LAN

  - 802.5 (Token Ring): Rare, ring topology

- Wireless LANs:

  - Use CSMA/CA and RTS/CTS mechanism to avoid collisions

## 8. Bridges

- Function: Connects two LAN segments to reduce traffic and collisions

- Types:

  - Transparent Bridge – Learns MAC addresses automatically

  - Source Routing Bridge – Uses routing info from source

Example:

- Two office floors connected via a bridge; reduces broadcast traffic on each floor

## 9. Summary Table

| Topic | Key Points | Example |
| --- | --- | --- |
| Framing | Byte stuffing, bit stuffing | HDLC |
| Error Detection | Parity, Checksum, CRC | Ethernet CRC-32 |
| Protocols | Stop-and-wait, Sliding Window | Go-Back-N ARQ |
| MAC Protocols | ALOHA, CSMA/CD, CSMA/CA | Ethernet, Wi-Fi |
| LAN/WLAN | IEEE 802.x | 802.3 Ethernet, 802.11 Wi-Fi |
| Bridges | Connect LAN segments, reduce collisions | Office floor networks |

### 10. Practice Problems

- Explain bit stuffing with an example frame.

- Compare Stop-and-Wait and Sliding Window protocols.

- What is CRC, and how does it detect errors?

- Explain the difference between CSMA/CD and CSMA/CA.

- Describe the role of a bridge in LANs.

# UNIT III – NETWORK LAYER

## 1. Network Layer – Overview

- The network layer is responsible for logical addressing, routing, and forwarding packets between source and destination across multiple networks.

- It works above the Data Link Layer and below the Transport Layer in the OSI model.

Functions:

- Logical Addressing: Assigns unique IP addresses to devices.

- Routing: Determines the path from source to destination.

- Forwarding: Moves packets through routers to the next hop.

- Fragmentation & Reassembly: Splits large packets to fit the link MTU and reassembles at the destination.

- Congestion & Flow Control: Helps avoid network congestion by managing traffic.

Example: Sending an email from New York to London involves multiple routers. The network layer decides the best path for the email packets.

## 2. Subnet Types

### 2.1 Virtual Circuit Subnets

- Connection-oriented: A pre-established path (virtual circuit) is created before sending data.

- Characteristics:

  - Packets follow the same path.

  - Each packet has a virtual circuit identifier (VCI).

- Advantages: Predictable delays, reliable ordering.

- Disadvantages: Setup required, less flexible.

Example: ATM (Asynchronous Transfer Mode) networks use virtual circuits for voice and video.

### 2.2 Datagram Subnets

- Connectionless: Each packet is treated independently.

- Characteristics:

  - Each packet carries full destination address.

  - Routes may differ per packet.

- Advantages: Flexible, no setup required.

- Disadvantages: Packets may arrive out of order.

Example: Internet Protocol (IP) networks use datagram subnets; packets from the same email may take different routes.

## 3. Routing Algorithms

Routing decides the path from source to destination.

### 3.1 Shortest Path Routing

- Determines path with minimum cost (distance, delay, hops).

- Algorithm: Dijkstra’s algorithm or Bellman-Ford.

- Example: Router calculates shortest route to deliver packets using routing table metrics.

### 3.2 Flooding

- Sends each packet on all outgoing links except the source link.

- Advantages: Guaranteed delivery if network is connected.

- Disadvantages: High traffic, possible duplicates.

- Example: Emergency alert systems in ad hoc networks may use flooding.

### 3.3 Hierarchical Routing

- Divides network into regions or domains to reduce table size.

- Example: Internet uses Autonomous Systems (AS); routers within AS exchange info internally, while gateways manage inter-AS routing.

### 3.4 Broadcast and Multicast Routing

- Broadcast: Sends packet to all nodes.

- Multicast: Sends packet to a specific group of nodes.

- Example: Video streaming to multiple subscribers uses multicast; ARP uses broadcast.

### 3.5 Distance Vector Routing

- Each router maintains a table of distances to all nodes.

- Routers share tables with neighbors periodically.

- Example Algorithm: Bellman-Ford; used in RIP (Routing Information Protocol).

- Problem: Count-to-infinity issue can occur.

### 3.6 Dynamic Routing

- Routing tables update automatically based on network changes.

- Adaptive routing: Adjusts based on congestion or failure.

- Example Protocols: OSPF, BGP.

### 3.7 Rotary Routing for Mobility

- For mobile networks, handles device movement between subnets.

- Supports handoffs to maintain continuous connectivity.

- Example: Mobile IP ensures a phone keeps the same IP while roaming.

## 4. Network Layer in the Internet

- Internet: Uses datagram subnet approach.

- Key Protocols: IPv4, IPv6, ICMP.

- Functions: Routing, addressing, fragmentation, and delivery.

- Example: An IP packet from a home computer to a web server passes through multiple routers dynamically.

## 5. Network Layer in ATM Networks

- ATM Networks: Use virtual circuits and fixed-size 53-byte cells.

- Combines voice, video, and data on the same network.

- Ensures predictable delay and high-speed transmission.

Example: Video conferencing over ATM provides smooth streaming with minimal jitter.

## 6. Summary Table

| Topic | Key Points | Example |
| --- | --- | --- |
| Subnets | Virtual circuit: connection-oriented, Datagram: connectionless | ATM – VC, Internet – Datagram |
| Routing | Shortest path, Flooding, Hierarchical, Broadcast, Multicast | Dijkstra – Shortest path, RIP – Distance vector |
| Dynamic Routing | Automatic updates based on network conditions | OSPF, BGP |
| Mobility | Supports mobile nodes | Mobile IP |
| Internet | Datagram network, IP routing | Sending email over the Internet |
| ATM | Virtual circuits, 53-byte cells | Video conferencing |

## 7. Practice Problems

- Compare virtual circuit and datagram subnets with real-world examples.

- Explain shortest path routing using a simple 5-node network.

- Discuss broadcast vs multicast routing with network examples.

- Illustrate distance vector routing and identify a potential problem.

- Explain rotary routing for mobile networks.

- Explain why ATM uses virtual circuits instead of datagrams.

# UNIT IV – TRANSPORT LAYER

## 1. Transport Layer – Overview

- Provides end-to-end communication between applications running on different hosts.

- Ensures reliable data transfer, flow control, and error detection.

Applications:

- FTP (File Transfer Protocol): Large file transfer with reliability.

- HTTP/HTTPS: Web browsing uses TCP for reliable delivery.

- Email (SMTP, IMAP, POP3): Reliable delivery of messages.

## 2. Transport Services

| Feature | TCP | UDP | Applications |
| --- | --- | --- | --- |
| Connection | Connection-oriented | Connectionless | TCP: Web browsing, Email, FTP UDP: Online gaming, VoIP, Live streaming |
| Reliability | Yes | No | TCP: File transfers, banking transactions UDP: Video conferences, DNS queries |
| Flow Control | Yes | No | TCP: Ensures sender does not overwhelm receiver UDP: Real-time traffic |

## 3. Connection Management

TCP Connection Setup: 3-Way Handshake

- Establishes a reliable session between client and server.

Applications:

- Opening a website (HTTP request/response).

- Remote login via SSH or Telnet.

TCP Connection Termination: 4-way handshake ensures all data is delivered.

- Example: Ending a file transfer session over FTP.

## 4. TCP and UDP Protocols

### 4.1 TCP (Transmission Control Protocol)

Applications:

- Web Browsing (HTTP/HTTPS): Ensures web pages arrive correctly.

- Email (SMTP, IMAP, POP3): Guarantees complete delivery.

- File Transfer (FTP/SFTP): Reliable transmission of large files.

### 4.2 UDP (User Datagram Protocol)

Applications:

- Online Gaming: Speed is critical; minor packet loss is tolerable.

- Video/Voice Streaming: Real-time delivery is prioritized over reliability.

- DNS Queries: Quick responses without connection overhead.

## 5. Congestion Control Algorithms

Applications:

- TCP Slow Start & Congestion Avoidance: Ensures network stability during large file transfers or streaming services.

- Fast Retransmit & Recovery: Used in web servers and cloud services to quickly handle packet loss.

Example: Netflix uses adaptive streaming with TCP to adjust video quality based on network congestion.

## 6. AAL Layer Protocol (ATM Adaptation Layer)

- ATM (Asynchronous Transfer Mode) is often used in high-speed networks.

- Provides segmentation and reassembly for voice, video, and data.

Applications:

- Video Conferencing: Real-time, smooth streaming using AAL1/AAL2.

- Voice Over ATM (VoATM): Reliable, low-latency voice transmission.

- IP over ATM: Data networks using AAL5 for packetized IP traffic.

## 7. Summary Table (With Applications)

| Topic | Key Points | Applications |
| --- | --- | --- |
| Transport Layer | End-to-end communication, segmentation, error control | FTP, HTTP, Email |
| TCP | Reliable, connection-oriented, flow control | Web browsing, Banking, FTP |
| UDP | Fast, connectionless | Online gaming, Video streaming, DNS |
| Connection Management | 3-way handshake for setup, 4-way termination | SSH, HTTP sessions, FTP |
| Congestion Control | Prevent network overload, TCP algorithms | Netflix, Cloud file transfers |
| AAL Layer | Segmentation & reassembly in ATM | Video conferencing, VoATM, IP networks |

## 8. Practice Problems (With Applications)

- Compare TCP and UDP and give real-world applications.

- Explain 3-way handshake in TCP with an example of opening a web page.

- Describe slow start and congestion avoidance in TCP and how it affects video streaming.

- Explain the role of AAL in ATM networks and give examples of real-time applications.

- List 2 advantages of TCP over UDP with application scenarios.

# UNIT V – APPLICATION LAYER

## 1. Overview of Application Layer

- The Application Layer is the topmost layer (Layer 7) of the OSI model.

- Provides interfaces for applications to access network services.

- Responsible for data formatting, encryption, and communication with end-user applications.

Functions:

- Network virtual terminal for remote access.

- File transfer, email, and directory services.

- Remote management using SNMP.

- World Wide Web (WWW) services using HTTP/HTTPS.

Example: Web browsers, email clients, and DNS resolvers operate at the application layer.

## 2. Domain Name System (DNS)

- Purpose: Translate human-readable domain names (e.g., www.google.com) into IP addresses.

- Structure: Hierarchical with multiple levels: root, top-level domains (TLD), second-level domains, and hostnames.

- Operation: Recursive and iterative query processes.

Example:

- Typing www.example.com in a browser triggers a DNS query to resolve it to an IP address like 192.168.1.1.

Applications:

- Browsing websites, email routing, cloud services.

## 3. Simple Network Management Protocol (SNMP)

- Purpose: Monitor and manage network devices like routers, switches, and servers.

- Components:

  - Manager: Central system that monitors devices.

  - Agent: Device that reports status.

  - MIB (Management Information Base): Database of device parameters.

Operations:

- Get, Set, Trap (notifications for critical events).

Example:

- A network administrator uses SNMP to monitor bandwidth usage or detect a failed router.

Applications:

- Network monitoring, fault detection, performance management.

## 4. Electronic Mail (Email)

### 4.1 SMTP – Simple Mail Transfer Protocol

- Purpose: Sending email from client to server or between servers.

- Port: 25 (default).

- Example: Sending an email from Gmail to Yahoo.

### 4.2 POP3 – Post Office Protocol 3

- Purpose: Download emails from server to local client.

- Port: 110 (default).

- Characteristic: Deletes emails from server after download (unless configured).

### 4.3 IMAP – Internet Message Access Protocol

- Purpose: Access emails directly on the server without downloading.

- Port: 143 (default).

- Characteristic: Keeps emails synchronized across devices.

### 4.4 MIME – Multipurpose Internet Mail Extensions

- Purpose: Allows email to carry multimedia (images, audio, video, attachments).

- Example: Sending an email with PDF attachments or images.

## 5. The World Wide Web (WWW)

- Purpose: Access and share information over the Internet using web pages.

- Components:

  - Web Browser: Client software (Chrome, Firefox).

  - Web Server: Hosts web pages (Apache, Nginx).

  - URL: Uniform Resource Locator for identifying resources.

Example: Visiting https://www.wikipedia.org displays content served by a web server.

Applications:

- Browsing, online shopping, cloud-based applications, online banking.

## 6. HTTP – Hypertext Transfer Protocol

- Purpose: Protocol for transferring web pages between client and server.

- Type: Application layer protocol.

- Methods:

  - GET: Retrieve data from server.

  - POST: Send data to server.

  - PUT, DELETE, PATCH: Modify or remove data.

- HTTP/HTTPS: HTTPS uses TLS/SSL for secure communication.

Example:

- Submitting a login form on a website uses POST over HTTPS.

Applications:

- Web browsing, online forms, REST APIs, cloud services.

## 7. Summary Table

| Topic | Key Points | Applications |
| --- | --- | --- |
| DNS | Resolves domain names to IP addresses | Browsing websites, email routing |
| SNMP | Network monitoring and management | Bandwidth monitoring, fault detection |
| SMTP | Sending emails | Sending emails from clients |
| POP3 | Download emails to client | Email clients like Outlook, Thunderbird |
| IMAP | Access emails on server | Gmail, Office 365, mobile email apps |
| MIME | Supports multimedia attachments | Sending PDFs, images, audio via email |
| WWW | Accessing information on Internet | Browsing, e-commerce, online banking |
| HTTP/HTTPS | Web page communication | Browsers, REST APIs, secure transactions |

## 8. Practice Problems

- Explain how DNS resolves a website name to an IP address.

- Compare POP3 and IMAP with examples.

- Describe the role of SNMP in network management.

- Explain how HTTP GET and POST methods work with examples.

- Discuss the role of MIME in email communication.

- Describe the components of WWW and their interactions.