# UNIT I: MOLECULAR STRUCTURE AND THEORIES OF BONDING

## 1. Introduction

Chemical bonding is the force that holds atoms together in molecules and solids. The study of molecular structure helps us understand:

- Molecular geometry and shape

- Bond strength and bond length

- Physical and chemical properties of substances

- Electrical, optical, and thermal properties of materials

Types of chemical bonding:

- Ionic Bonding: Transfer of electrons; forms lattice structures; e.g., NaCl

- Covalent Bonding: Sharing of electrons; directional; e.g., H2, CH4

- Metallic Bonding: Delocalized electrons; explains conductivity in metals

- Van der Waals Forces: Weak interactions; e.g., noble gas solids

## 2. Atomic and Molecular Orbitals

### 2.1 Atomic Orbitals (AO)

- Atomic orbitals represent regions of high probability of finding electrons.

- Types: s, p, d, f orbitals

- Quantum numbers: n (principal), l (angular momentum), m_l (magnetic), m_s (spin)

Key points:

- s orbital: spherical, one per energy level

- p orbital: dumbbell-shaped, three per energy level

- d orbital: cloverleaf-shaped, five per energy level

### 2.2 Molecular Orbitals (MO)

- Molecular orbitals are formed by combining atomic orbitals using the LCAO method.

- Electrons in MOs are delocalized over the molecule.

- Types:

  - Bonding MO (σ, π) → lowers energy, stabilizes molecule

  - Antibonding MO (σ*, π*) → higher energy, destabilizes molecule

Example: H2 molecule:

σ1s = 1s_A + 1s_B (bonding)

σ*1s = 1s_A - 1s_B (antibonding)

## 3. Linear Combination of Atomic Orbitals (LCAO)

The molecular orbital ψ_MO is expressed as:

ψ_MO = c1*ψ_A + c2*ψ_B

- ψ_A and ψ_B = atomic orbitals

- c1, c2 = coefficients (weights)

- Conditions for effective combination:

  - Orbitals must have similar energy

  - Orbitals must have proper symmetry

Example: Formation of σ and σ* orbitals in H2:

- σ = constructive combination

- σ* = destructive combination

## 4. Energy Level Diagrams of Diatomic Molecules

### 4.1 Homonuclear Diatomic Molecules (N2, O2, F2)

- N2: 10 valence electrons → bond order = 3 → triple bond → diamagnetic

- O2: 12 valence electrons → bond order = 2 → double bond → paramagnetic

- F2: 14 valence electrons → bond order = 1 → single bond → diamagnetic

MO Energy Order (for Z ≤ 8):

σ1s < σ*1s < σ2s < σ*2s < σ2pz < π2px ≈ π2py < π*2px ≈ π*2py < σ*2pz

### 4.2 Heteronuclear Diatomic Molecules (CO, HF)

- CO: bond order = 3, strong triple bond, electrons slightly shifted toward O

- HF: bond order = 1, highly polar bond

## 5. π-Molecular Orbitals in Conjugated Systems

### 5.1 Butadiene (CH2=CH-CH=CH2)

- 4 π electrons delocalized

- Forms 4 π molecular orbitals: ψ1 < ψ2 < ψ3 < ψ4

- Bonding: ψ1, ψ2; Antibonding: ψ3, ψ4

Example: Energy splitting explains UV absorption in conjugated dienes

### 5.2 Benzene (C6H6)

- 6 π electrons delocalized → 3 bonding π-MOs, 3 antibonding π-MOs

- Explains aromatic stability

- Hückel rule: aromatic if 4n+2 π electrons (n=1 → 6 electrons)

Example: Benzene’s stability is higher than hypothetical cyclohexatriene

## 6. Bonding in Solids

### 6.1 Band Theory of Solids

- Atomic orbitals in solids overlap → energy bands

- Valence band: filled with electrons

- Conduction band: empty, allows electron movement

- Band gap (Eg): energy difference between conduction and valence band

Types of solids based on band gap:

| Solid type | Band gap (eV) | Example |
| --- | --- | --- |
| Conductor | 0 | Cu, Ag |
| Semiconductor | 0.1 – 3 | Si, Ge |
| Insulator | > 3 | Diamond |

### 6.2 Effect of Doping

- n-type doping: adds electrons (donor) → conductivity increases

- p-type doping: creates holes (acceptor) → conductivity increases

Example:

- Silicon doped with Phosphorus → n-type

- Silicon doped with Boron → p-type

## 7. Additional Topics

### 7.1 Bond Order

Bond Order = (Number of bonding electrons – Number of antibonding electrons)/2

- N2: Bond order = 3 → triple bond

- O2: Bond order = 2 → double bond

### 7.2 Magnetic Properties

- Paramagnetic: unpaired electrons

- Diamagnetic: all electrons paired

### 7.3 Aromaticity

- Hückel rule: 4n + 2 π electrons → aromatic

- Benzene: 6 π electrons → aromatic, planar, cyclic

### 7.4 Conjugation and Stability

- Delocalized π electrons → lower energy → stability

- Examples: Butadiene, Benzene

## 8. Practice Examples

- Draw MO diagram of O2, predict bond order and magnetic nature.

- Determine bond order for CO and HF.

- Draw π-MO diagram for butadiene and benzene.

- Explain aromaticity using Hückel’s rule.

- Explain effect of n-type and p-type doping in silicon.

## 9. Summary

- Chemical bonding explains molecular stability and properties

- Molecular orbitals give information about bond order, magnetism

- Conjugation and aromaticity explain stability of organic molecules

- Band theory explains electronic properties of solids

- Doping modifies conductivity

This version now includes:

- Expanded theory on all topics (bonding, MO theory, π-MOs, band structure, doping)

- Examples for N2, O2, F2, CO, HF, butadiene, benzene

- Tables, formulas, and important points

- Practice problems

# UNIT II: WATER AND ITS TREATMENT

## 1. Hardness of Water

### 1.1 Definition

Hardness is the measure of the concentration of divalent metal ions, primarily Ca²⁺ and Mg²⁺, in water.

- Importance: Hard water causes scaling in boilers, reduces soap efficiency, and affects industrial processes.

### 1.2 Types of Hardness

- Temporary Hardness

  - Caused by bicarbonates: Ca(HCO₃)₂, Mg(HCO₃)₂

  - Can be removed by boiling

  - Reaction:

  - Ca(HCO₃)₂ → CaCO₃ (precipitate) + CO₂ + H₂O

  - Mg(HCO₃)₂ → Mg(OH)₂ (precipitate) + CO₂

- Permanent Hardness

  - Caused by sulfates and chlorides: CaSO₄, MgCl₂

  - Cannot be removed by boiling

- Total Hardness

  - Sum of temporary and permanent hardness

  - Expressed as mg/L of CaCO₃

### 1.3 Measurement of Hardness (Complexometric Method)

EDTA Titration Method

- Indicator: Eriochrome Black T

- Reaction: EDTA forms stable complexes with Ca²⁺ and Mg²⁺

Procedure:

- Take a measured water sample (e.g., 50 mL)

- Add buffer solution (pH 10) to maintain optimal EDTA activity

- Add Eriochrome Black T → wine red color

- Titrate with standard EDTA solution until color changes to blue

Calculation of Hardness:

Hardness (mg/L as CaCO₃) = (Volume of EDTA × Molarity × 50,000) / Volume of water sample (mL)

Example:

- Sample volume = 50 mL

- EDTA used = 25 mL, Molarity = 0.01 M

Hardness = (25 × 0.01 × 50,000)/50 = 250 mg/L as CaCO₃

Practical Note: Total hardness guides boiler water treatment and domestic water usage.

## 2. Potable Water and Its Specifications

### 2.1 Potable Water

- Water safe for human consumption

- Must be free from pathogens, harmful chemicals, and excessive salts

### 2.2 Specifications (WHO / IS 10500)

| Parameter | Limit | Importance |
| --- | --- | --- |
| pH | 6.5 – 8.5 | Neutral to slightly alkaline |
| Hardness | ≤ 300 mg/L | Prevents scaling and soap wastage |
| Chlorides | ≤ 250 mg/L | Avoids taste and corrosion |
| Sulfates | ≤ 200 mg/L | Avoids laxative effect |
| Iron | ≤ 0.3 mg/L | Prevents taste, staining, and deposits |
| Bacteria | None in 100 mL | Safe from diseases like cholera |

Importance: Ensures health safety and prevents infrastructure damage.

## 3. Treatment of Potable Water

Water treatment ensures removal of impurities, pathogens, and hardness.

### 3.1 Steps Involved

- Screening: Removal of large debris, leaves, and floating materials

- Coagulation and Flocculation:

  - Add alum (Al₂(SO₄)₃) → forms flocs

  - Flocs trap fine particles and suspended solids

- Sedimentation: Flocs settle to form sludge

- Filtration: Water passes through sand or activated carbon filters

- Disinfection: Killing of microbes using chlorination, ozone, or UV

### 3.2 Disinfection by Chlorination

Chlorination:

- Cl₂ added to water reacts to form hypochlorous acid (HOCl)

- HOCl kills bacteria and viruses

Break-point chlorination:

- Chlorine added until all ammonia, nitrogen compounds, and bacteria are oxidized

- Ensures complete disinfection without excess free chlorine

Reaction:

Cl₂ + H₂O → HOCl + HCl

HOCl → OCl⁻ + H⁺ (active disinfectant)

Example:

- 1 mg/L chlorine kills 99.9% bacteria in 30 minutes

## 4. Boiler Troubles

### 4.1 Sludges

- Deposits of insoluble salts (CaCO₃, Mg(OH)₂)

- Cause reduced efficiency and overheating

### 4.2 Scales

- Hard deposits on boiler walls

- Cause poor heat transfer, increased fuel consumption

### 4.3 Caustic Embrittlement

- Cracks in boiler metal caused by concentrated NaOH

- Occurs due to improper internal treatment

## 5. Internal Treatment of Boiler Feed Water

Purpose: Prevent scale and corrosion inside the boiler

### 5.1 Calgon Conditioning

- Sodium hexametaphosphate added to bind Ca²⁺ and Mg²⁺ ions

- Forms soluble complexes → prevents scale deposition

### 5.2 Phosphate Conditioning

- Sodium phosphate (Na₃PO₄) reacts with calcium:

3Ca²⁺ + 2PO₄³⁻ → Ca₃(PO₄)₂ (soft sludge)

- Soft sludge removed by blowdown

### 5.3 Colloidal Conditioning

- Organic polymers disperse colloidal particles

- Prevents formation of scale and sludge

## 6. External Treatment Methods

### 6.1 Ion-Exchange Softening

- Removes Ca²⁺ and Mg²⁺ ions from water

- Cation-exchange resin: replaces Ca²⁺/Mg²⁺ with Na⁺

Ca²⁺ + 2Na⁺(resin) → Ca²⁺(resin) + 2Na⁺(water)

### 6.2 Lime-Soda Softening

- Adds Ca(OH)₂ + Na₂CO₃ to precipitate hardness salts

Ca²⁺ + CO₃²⁻ → CaCO₃ ↓

Mg²⁺ + OH⁻ → Mg(OH)₂ ↓

## 7. Desalination of Water

### 7.1 Reverse Osmosis (RO)

- Water forced through semipermeable membrane under pressure

- Removes salts, minerals, and contaminants

- Produces high-quality water for drinking and industrial use

### 7.2 Other Methods

- Distillation: Boiling and condensing water

- Electrodialysis: Uses electric field to remove ions

Applications: Drinking water, industries, boilers, laboratories

## 8. Summary

- Hardness is caused by Ca²⁺ and Mg²⁺ ions

- EDTA titration measures total hardness

- Potable water must meet WHO / IS 10500 standards

- Treatment involves coagulation, sedimentation, filtration, and disinfection

- Boilers require internal treatment (Calgon, phosphate, colloidal)

- External treatment softens water using ion exchange or lime-soda

- Desalination removes salts, mainly by RO or distillation

## 9. Practice Numerical Problems

- 50 mL water requires 15 mL of 0.01 M EDTA. Find hardness.

Hardness = (15 × 0.01 × 50,000)/50 = 150 mg/L as CaCO₃

- Water contains 80 mg/L Ca²⁺ and 60 mg/L Mg²⁺. Calculate total hardness.

Total Hardness = 80 + 60 = 140 mg/L

- Explain why break-point chlorination is preferred over simple chlorination.

- Describe internal and external treatment methods for boiler water.

# UNIT III: BATTERY CHEMISTRY & CORROSION

## 1. Battery Chemistry

### 1.1 Introduction

A battery is a device that converts chemical energy into electrical energy through redox reactions. Batteries are widely used in portable electronics, vehicles, and industrial applications.

Key points:

- Batteries store energy chemically

- Provide a steady voltage and current

- Can be primary (non-rechargeable) or secondary (rechargeable)

### 1.2 Classification of Batteries

- Primary Batteries – Non-rechargeable, cannot be regenerated once discharged

  - Examples: Zinc-Carbon, Alkaline batteries, Zinc-Air batteries

- Secondary Batteries – Rechargeable, can be reused multiple times

  - Examples: Lead-acid battery, Nickel-Cadmium (Ni-Cd), Lithium-ion batteries

- Reserve Batteries – Stored inactive; activated when needed

  - Examples: Air-zinc batteries, military applications

### 1.3 Basic Requirements for Commercial Batteries

Commercial batteries must satisfy the following:

- High energy density

- High efficiency and low self-discharge

- Good cycle life (number of charge-discharge cycles)

- Safe operation and environmentally friendly

- Cost-effective and suitable for mass production

## 2. Important Batteries

### 2.1 Zinc-Air Battery

- Type: Primary battery

- Construction:

  - Zinc anode

  - Air cathode (O₂)

  - Electrolyte: KOH or NaOH

- Working:

  - Oxygen from air reacts at cathode

  - Zinc oxidizes at anode: Zn → Zn²⁺ + 2e⁻

  - Electrons flow through external circuit → electricity

- Applications:

  - Hearing aids

  - Small electronic devices

  - Backup power systems

Example Reaction:

Anode: Zn + 2OH⁻ → Zn(OH)₂ + 2e⁻

Cathode: O₂ + 2H₂O + 4e⁻ → 4OH⁻

Overall: 2Zn + O₂ → 2ZnO

### 2.2 Lithium-Ion Battery

- Type: Secondary (Rechargeable) battery

- Construction:

  - Anode: Graphite

  - Cathode: LiCoO₂ or LiFePO₄

  - Electrolyte: Lithium salts in organic solvents

- Working:

  - During discharge: Li⁺ ions move from anode → cathode

  - Electrons flow through external circuit → power devices

  - During charging: Process reversed

Applications:

- Smartphones, laptops

- Electric vehicles (EVs)

- Renewable energy storage systems

Advantages:

- High energy density

- Low self-discharge

- Lightweight

### 2.3 Fuel Cells

Fuel cells convert chemical energy directly into electrical energy without combustion.

#### 2.3.1 Differences between Battery and Fuel Cell

| Parameter | Battery | Fuel Cell |
| --- | --- | --- |
| Energy Source | Stored in battery | Continuous fuel supply |
| Duration | Limited | Long-lasting |
| Charging | Rechargeable (if secondary) | Not needed |
| Application | Portable electronics | Power generation, vehicles |

#### 2.3.2 Methanol-Oxygen Fuel Cell

- Fuel: Methanol (CH₃OH)

- Oxidant: Oxygen (O₂)

- Reaction:

Anode: CH₃OH + H₂O → CO₂ + 6H⁺ + 6e⁻

Cathode: 3/2 O₂ + 6H⁺ + 6e⁻ → 3 H₂O

Overall: CH₃OH + 3/2 O₂ → CO₂ + 2 H₂O

- Applications: Portable power sources, electric vehicles

#### 2.3.3 Solid Oxide Fuel Cell (SOFC)

- Electrolyte: Solid oxide ceramic (ZrO₂ doped with Y₂O₃)

- Operating temperature: 600–1000°C

- Fuel: Hydrogen or hydrocarbons

- Advantages: High efficiency, can use renewable fuels

- Applications: Stationary power plants, auxiliary power for vehicles

### 2.4 Solar Cells

- Convert sunlight into electricity using the photovoltaic effect

- Material: Silicon (monocrystalline, polycrystalline)

- Applications:

  - Rooftop power generation

  - Satellites and space probes

  - Remote areas with no grid

Example: Single-junction silicon solar cell has efficiency ~15–20%

## 3. Corrosion

### 3.1 Introduction

Corrosion is the deterioration of metals due to chemical or electrochemical reactions with the environment.

Importance:

- Leads to structural damage, economic loss, and safety hazards

- Affects pipelines, bridges, boilers, and vehicles

### 3.2 Causes and Effects

- Causes: Moisture, oxygen, salts, acids, temperature, microbial activity

- Effects: Weakening of metal, failure of structures, contamination

### 3.3 Theories of Corrosion

- Chemical Corrosion Theory: Direct reaction of metal with environment

  - Example: Fe + 1/2 O₂ + H₂O → Fe²⁺ + 2OH⁻

- Electrochemical Corrosion Theory: Localized electrochemical cells form on metal surface

  - Metal acts as anode and cathode

### 3.4 Mechanism of Electrochemical Corrosion

- Formation of anodic and cathodic sites on the metal surface

- Anodic reaction: Metal oxidized to ions

- Cathodic reaction: Oxygen reduction in water

- Electron flow through metal, ion flow through electrolyte

Example (Iron in water):

Anode: Fe → Fe²⁺ + 2e⁻

Cathode: O₂ + 4e⁻ + 2H₂O → 4OH⁻

Overall: 2Fe + O₂ + 2H₂O → 2Fe(OH)₂

### 3.5 Types of Corrosion

- Galvanic Corrosion – Occurs when two dissimilar metals are in contact in an electrolyte

- Water-line Corrosion – Occurs at the interface of air and water on submerged metals

- Pitting Corrosion – Localized corrosion creating holes or pits

### 3.6 Factors Affecting Corrosion Rate

- Nature of metal and alloy

- Presence of electrolytes (salts, acids)

- Oxygen concentration and temperature

- pH of the environment

- Presence of microorganisms

### 3.7 Corrosion Control Methods

#### 3.7.1 Cathodic Protection

- Makes the metal cathode to prevent oxidation

- Two methods:

- Sacrificial Anode:

  - More reactive metal (Zn, Mg) corrodes instead of the protected metal

  - Example: Zinc anode protects iron pipelines

- Impressed Current:

  - External DC power source supplied to make metal cathodic

  - Used in pipelines, storage tanks, ships

#### 3.7.2 Other Methods

- Coatings: Paints, plastics, galvanization

- Alloying metals: Stainless steel

- Corrosion inhibitors: Chemicals added to water to reduce metal oxidation

## 4. Summary Table

| Topic | Key Points |
| --- | --- |
| Batteries | Primary, secondary, reserve; commercial requirements |
| Zn-Air Battery | Primary, uses oxygen, applications: hearing aids |
| Li-Ion Battery | Rechargeable, EVs, portable electronics |
| Fuel Cells | Continuous energy supply; Methanol-Oxygen & SOFC examples |
| Solar Cells | Photovoltaic effect, applications in renewable energy |
| Corrosion | Chemical/electrochemical deterioration of metals |
| Types of Corrosion | Galvanic, water-line, pitting |
| Control Methods | Cathodic protection, sacrificial anode, impressed current, coatings |

## 5. Practice Problems & Questions

- Explain the working of a Li-ion battery and its use in electric vehicles.

- Compare battery vs fuel cell in terms of energy supply and efficiency.

- Write reactions for Zn-Air and Methanol-Oxygen fuel cells.

- Explain galvanic corrosion with an example.

# UNIT IV: ENERGY SOURCES

## 1. Introduction

Energy is essential for industrial, domestic, and transportation purposes. Energy sources can be primary fuels (coal, petroleum, natural gas, biomass) or secondary fuels (electricity, hydrogen).

Importance of energy sources:

- Power generation

- Transportation

- Heating and industrial processes

- Sustainable development

## 2. Calorific Value of Fuel

Definition:
Calorific value (CV) is the amount of heat released per unit mass or volume of fuel during complete combustion.

- Unit: kcal/kg or MJ/kg

Types:

- Higher Calorific Value (HCV or Gross Calorific Value)

  - Includes latent heat of vaporization of water formed during combustion

- Lower Calorific Value (LCV or Net Calorific Value)

  - Excludes latent heat of water vapor

Relation:

LCV = HCV – Heat of vaporization of water

### 2.1 Dulong’s Formula

Dulong’s formula estimates HCV of a solid or liquid fuel based on its composition:

HCV (kcal/kg) = 81.0*C + 300* (H – O/8) + 25*S

Where:

- C = % Carbon

- H = % Hydrogen

- O = % Oxygen

- S = % Sulfur

Example:

- Coal contains 70% C, 5% H, 10% O, 1% S

HCV = 81*70 + 300*(5 – 10/8) + 25*1

= 5670 + 300*(3.75) + 25

= 5670 + 1125 + 25

= 6820 kcal/kg

## 3. Solid Fuels: Coal

### 3.1 Introduction

- Coal is a fossil fuel formed from decayed plant matter over millions of years.

- Widely used for thermal power plants, industries, and metallurgy.

### 3.2 Analysis of Coal

- Proximate Analysis: Determines moisture, volatile matter, ash, and fixed carbon

  - Moisture (M): Water content

  - Volatile matter (VM): Organic gases released on heating

  - Ash (A): Residue after complete combustion

  - Fixed Carbon (FC): Remaining combustible matter

Significance: Determines combustion efficiency and heating value

- Ultimate Analysis: Determines elemental composition: C, H, O, S, N

  - Helps calculate calorific value and combustion characteristics

Example: Coal with C=70%, H=5%, O=10%, S=1%, N=2% → Used in Dulong formula

## 4. Liquid Fuels: Petroleum

### 4.1 Petroleum

- A mixture of hydrocarbons obtained from crude oil

- Used as fuel in engines, industries, and domestic purposes

### 4.2 Petroleum Refining

- Distillation: Separates crude oil into fractions based on boiling point

- Cracking: Breaks long-chain hydrocarbons into lighter, more useful fractions

  - Thermal Cracking: High temperature and pressure

  - Catalytic Cracking: Uses catalysts at lower temperatures

Example: Heavy oil → gasoline, diesel

### 4.3 Knocking in Engines

- Knocking: Premature ignition of fuel in engines causing noise and engine damage

- Prevention: Using fuels with higher octane number

Ratings:

- Octane Rating: For gasoline, higher octane → less knocking

- Cetane Rating: For diesel, higher cetane → better ignition quality

### 4.4 Synthetic Petrol (Fischer-Tropsch Process)

- Converts coal or biomass into liquid hydrocarbons

- Uses CO and H₂ over iron or cobalt catalyst

- Produces synthetic fuels like petrol and diesel

n CO + (2n+1) H₂ → CnH2n+2 + n H₂O

## 5. Gaseous Fuels

### 5.1 Natural Gas

- Mainly methane (CH₄)

- High calorific value, clean burning

### 5.2 Liquefied Petroleum Gas (LPG)

- Mixture of propane (C₃H₈) and butane (C₄H₁₀)

- Used for cooking, heating, and vehicles

### 5.3 Compressed Natural Gas (CNG)

- Compressed CH₄ stored at high pressure

- Advantages: Clean, low CO₂ emission, cheaper than petrol/diesel

Applications of Gaseous Fuels:

- Domestic cooking

- Industrial heating

- Automotive fuel

## 6. Biofuels: Biodiesel

### 6.1 Introduction

- Renewable fuel made from vegetable oils or animal fats

- Alternative to diesel

### 6.2 Production: Transesterification

- Triglycerides react with methanol/ethanol in the presence of NaOH or KOH catalyst

Triglyceride + 3 MeOH → 3 Methyl esters (Biodiesel) + Glycerol

### 6.3 Advantages

- Renewable and eco-friendly

- Reduces CO and particulate emissions

- Can be used in existing diesel engines

## 7. Summary Table

| Topic | Key Points |
| --- | --- |
| Calorific Value | HCV, LCV, Dulong’s formula |
| Solid Fuels (Coal) | Proximate and ultimate analysis, significance |
| Liquid Fuels (Petroleum) | Refining, cracking, knocking, octane & cetane rating, Fischer-Tropsch process |
| Gaseous Fuels | Natural gas, LPG, CNG: composition, applications |
| Biofuels (Biodiesel) | Transesterification, advantages |

## 8. Practice Problems

- Calculate HCV of a coal sample: C=75%, H=5%, O=10%, S=1% using Dulong formula.

- Explain thermal vs catalytic cracking with examples.

- Compare Octane and Cetane ratings for fuels.

- Write the Fischer-Tropsch reaction for synthetic petrol.

- Explain the production of biodiesel by transesterification.

# UNIT V: POLYMERIC MATERIALS

## 1. Introduction to Polymers

### 1.1 Definition

A polymer is a large molecule (macromolecule) formed by repetition of small units called monomers.

Example:

- Polyethylene (PE): Monomer = ethylene (CH₂=CH₂)

### 1.2 Classification of Polymers

| Type | Description | Examples |
| --- | --- | --- |
| Natural Polymers | Found in nature | Cellulose, Natural Rubber, Proteins |
| Synthetic Polymers | Man-made polymers | Nylon, PVC, Bakelite, Teflon |
| Organic Polymers | Carbon-based backbone | Polyethylene, Polystyrene |
| Inorganic Polymers | Non-carbon backbone | Silicones, Boron Nitride Polymers |

### 1.3 Types of Polymerization

- Addition (Chain-Growth) Polymerization

  - Monomers add together without loss of small molecules

  - Usually unsaturated monomers (with double bonds)

  - Example: Polyethylene, Polypropylene

Mechanism (Free Radical Addition):

  - Initiation: Formation of free radicals

  - R-O-O-R → 2 R-O•

  - Propagation: Radical reacts with monomer to form chain

  - R• + CH₂=CH₂ → R-CH₂-CH₂•

  - Termination: Two radicals combine

  - R-CH₂-CH₂• + •CH₂-CH₂-R → R-CH₂-CH₂-CH₂-CH₂-R

- Condensation (Step-Growth) Polymerization

  - Monomers react to form polymer with elimination of small molecules (H₂O, HCl, etc.)

  - Example: Nylon 6:6, Polyester (PET)

Example – Nylon 6:6 Formation:

Hexamethylene diamine + Adipic acid → Nylon 6:6 + H₂O

H₂N-(CH₂)₆-NH₂ + HOOC-(CH₂)₄-COOH → [-NH-(CH₂)₆-NH-CO-(CH₂)₄-CO-]n + H₂O

## 2. Plastics

### 2.1 Definition

Plastics are synthetic polymers that can be molded into shapes.

### 2.2 Characteristics

- Lightweight, durable, and versatile

- Resistant to corrosion and chemicals

- Can be thermoplastic or thermosetting

### 2.3 Types of Plastics

- Thermoplastics – Soften on heating, harden on cooling (reversible)

  - Example: PVC, Polyethylene, Polystyrene

- Thermosetting Plastics – Hard and rigid after curing, cannot be remolded

  - Example: Bakelite, Teflon

### 2.4 Important Plastics

- PVC (Polyvinyl Chloride)

  - Monomer: Vinyl chloride (CH₂=CHCl)

  - Preparation: Free radical polymerization

  - Properties: Lightweight, chemically resistant, electrically insulating

  - Applications: Pipes, cable insulation, window frames

- Bakelite (Phenol-Formaldehyde Resin)

  - Type: Thermosetting plastic

  - Reaction:

  - Phenol + Formaldehyde → Bakelite + H₂O

  - Properties: Heat resistant, hard, insulating

  - Applications: Electrical switches, laminates, radio casings

- Teflon (PTFE)

  - Monomer: Tetrafluoroethylene (CF₂=CF₂)

  - Properties: High chemical resistance, non-stick, heat resistant

  - Applications: Non-stick cookware, chemical containers, gaskets

## 3. Rubbers and Elastomers

### 3.1 Natural Rubber

- Source: Latex from Hevea brasiliensis

- Composition: Polyisoprene

- Characteristics: Elastic, waterproof, soft

- Vulcanization: Heating with sulfur to form cross-links

  - Improves elasticity, strength, and durability

Polyisoprene + Sulfur → Cross-linked rubber

### 3.2 Synthetic Elastomers

- Buna-S Rubber (Styrene-Butadiene Rubber)

  - Monomers: Butadiene + Styrene

  - Properties: Heat resistant, wear-resistant

  - Applications: Tires, shoe soles

- Butyl Rubber (Isobutylene-Isoprene Rubber)

  - Monomers: Isobutylene + small amount of Isoprene

  - Properties: Impermeable to gases, flexible, chemical resistant

  - Applications: Inner tubes, seals, adhesives

## 4. Conducting Polymers

### 4.1 Introduction

- Conducting polymers are organic polymers that conduct electricity

- Unlike normal polymers, they have conjugated double bonds allowing electron mobility

### 4.2 Characteristics

- Electrical conductivity (metal-like or semiconducting)

- Light weight, flexible, and processable

- Can be doped to increase conductivity

### 4.3 Classification

| Type | Example |
| --- | --- |
| Intrinsically Conducting | Polyacetylene, Polyaniline |
| Extrinsically Conducting | PVC doped with iodine |

### 4.4 Mechanism: Conducting Polymer – Trans-Polyacetylene

- Structure: Linear chain of CH units with alternating single and double bonds

- Doping: Adding iodine (I₂) or bromine (Br₂) increases conductivity

[–CH=CH–]n + I₂ → Conducting polyacetylene

- Applications:

  - Organic LEDs (OLEDs)

  - Sensors

  - Batteries and supercapacitors

## 5. Summary Table

| Topic | Key Points |
| --- | --- |
| Polymers | Large molecules made from monomers |
| Polymerization | Addition (free radical), Condensation (Nylon 6:6) |
| Plastics | Thermoplastics (PVC), Thermosets (Bakelite, Teflon) |
| Rubbers & Elastomers | Natural rubber (vulcanization), Buna-S, Butyl rubber |
| Conducting Polymers | Trans-polyacetylene, doped polymers, electronic applications |

## 6. Practice Questions

- Explain the difference between thermoplastic and thermosetting plastics.

- Write the reaction for Nylon 6:6 formation.

- Describe vulcanization of natural rubber and its importance.

- Explain the mechanism of free radical polymerization.

- Discuss conducting polymers, their mechanism, and applications.

This version covers all topics of Unit V in full detail, including:

- Polymer types, polymerization reactions, mechanisms

- Plastics (PVC, Bakelite, Teflon) and their applications

- Natural and synthetic rubbers, elastomers (Buna-S, Butyl rubber)

- Conducting polymers, trans-polyacetylene mechanism, applications

- Includes tables, reactions, examples, and practice questions