# UNIT I – SEMICONDUCTOR PHYSICS & DIODES

Semiconductor devices form the backbone of modern electronics, including diodes, transistors, and integrated circuits.

## 1. Formation of P-type and N-type Semiconductors

### 1.1 Intrinsic Semiconductors

- Pure semiconductor materials like Silicon (Si) or Germanium (Ge).

- Conductivity is low because only a few electrons are free at room temperature.

### 1.2 Extrinsic Semiconductors

- Doping is done to increase conductivity.

- N-type:

  - Doped with pentavalent atoms (e.g., Phosphorus, Arsenic)

  - Extra electrons → majority carriers

  - Holes → minority carriers

- P-type:

  - Doped with trivalent atoms (e.g., Boron, Gallium)

  - Creates holes as majority carriers

  - Electrons → minority carriers

Diagram:

N-type: e- are majority

P-type: holes are majority

## 2. Principle and Operation of a Diode

- A PN junction diode allows current to flow in one direction only.

- Formed by joining P-type and N-type materials.

### 2.1 Forward Bias

- P connected to positive, N to negative terminal → current flows.

- Barrier potential (~0.7V for Si) must be overcome.

### 2.2 Reverse Bias

- P connected to negative, N to positive terminal → current is very small (reverse saturation current).

## 3. Volt-Ampere Characteristics

- Graph of current (I) vs voltage (V).

- Forward Bias Region: Current increases exponentially after threshold voltage.

- Reverse Bias Region: Current is very small (saturation current).

Example:

- Si diode threshold voltage: 0.7V

- Ge diode threshold voltage: 0.3V

## 4. Current Components in a PN Diode

Total diode current ( I ) consists of:

- Drift Current (I_drift) – due to minority carriers moving across junction

- Diffusion Current (I_diffusion) – due to majority carriers moving across junction

## 5. Diode Equation

The current through a diode is given by:

[
I = I_S \left( e^{\frac{V}{nV_T}} - 1 \right)
]

Where:

- ( I_S ) = Reverse saturation current

- ( V ) = Voltage across diode

- ( V_T ) = Thermal voltage ( = \frac{kT}{q} ) (~26 mV at 300K)

- ( n ) = Ideality factor (1 for ideal diode)

## 6. Temperature Dependence

- ( I_S ) increases with temperature → more minority carriers.

- Forward voltage decreases with temperature (~ -2 mV/°C for Si diode).

## 7. Static and Dynamic Resistances

- Static Resistance ( R_s )
[
R_s = \frac{V}{I} \text{ at a point on the V-I curve}
]

- Dynamic Resistance ( r_d )
[
r_d = \frac{\Delta V}{\Delta I} \text{ (slope of V-I curve)}
]

Example:

- Forward voltage ( V = 0.7V ), current ( I = 10mA )

- Static resistance: ( R_s = 0.7/0.01 = 70 \Omega )

## 8. Equivalent Circuit of Diode

- Forward-biased diode can be modeled as:

Series combination of:

- Ideal diode

- Series resistance ( r_s ) (accounts for bulk material resistance)

Diagram:

|----->|----R_s----|

## 9. Load Line Analysis

- Combines diode V-I characteristics with external circuit to find operating point (Q-point).

- Steps:

  - Plot diode V-I curve

  - Draw load line ( V = V_s - I R_L )

  - Intersection → Q-point

## 10. Diode Capacitances

- Diffusion Capacitance (( C_d ))

  - Due to charge storage in forward bias.

- Transition Capacitance (( C_t ))

  - Due to depletion region in reverse bias.
[
C_t = \frac{\epsilon A}{W}
]
Where ( W ) = depletion width

## 11. Breakdown Mechanisms in Diode

- Zener Breakdown – High reverse voltage → strong electric field → electrons tunnel → diode conducts.

- Avalanche Breakdown – High reverse voltage → collision of carriers → multiplication → diode conducts.

## 12. Zener Diode and Its Applications

- Operates in reverse breakdown region to provide voltage regulation.

Example Circuit: Voltage Regulator

V_in ----|>---- R ---- V_out

ZD

- V_out remains nearly constant even if V_in or load changes.

Applications:

- Voltage regulation, surge protection, waveform clipping.

## Summary Table – Diodes Concepts

| Topic | Description | Example/Note |
| --- | --- | --- |
| P-type / N-type | Doped semiconductors | P: holes, N: electrons |
| PN Junction Diode | One-way current device | Forward/Reverse bias |
| V-I Characteristics | Current vs voltage | Si: 0.7V threshold |
| Current Components | Drift + Diffusion | I_total = I_drift + I_diffusion |
| Diode Equation | I = I_S(e^(V/nV_T)-1) | Forward current formula |
| Static / Dynamic Resistance | Point vs slope | R_s = V/I, r_d = dV/dI |
| Diode Capacitances | Diffusion & Transition | C_d, C_t |
| Breakdown | Zener & Avalanche | High reverse voltage |
| Zener Diode Applications | Voltage regulation | Maintains constant V_out |

Key Points

- PN junction diode allows current in one direction only.

- Diode current depends on temperature and applied voltage.

- Load line and equivalent circuits help analyze real diode circuits.

- Zener diode is useful for voltage regulation and protection.

# UNIT II – RECTIFIERS AND FILTERS

Rectifiers are electronic circuits that convert AC (Alternating Current) to DC (Direct Current). Filters are used to smooth the pulsating DC output.

## 1. P-N Junction Diode as a Rectifier

- A diode allows current to flow in one direction only, making it suitable for rectification.

- Types of rectifiers:

  - Half Wave Rectifier

  - Full Wave Rectifier

  - Bridge Rectifier

## 2. Half Wave Rectifier

### Operation

- Uses one diode.

- Conducts only during the positive half-cycle of AC.

- Output is pulsating DC.

### Circuit Diagram

AC Source → Diode → Load Resistor (R_L)

### Key Points

- Only positive half-cycle appears across load.

- Negative half-cycle is blocked.

### Waveform

- Output current flows in pulses, one pulse per AC cycle.

### Efficiency

[
\eta = \frac{P_{dc}}{P_{ac}} \approx 40.6%
]

## 3. Full Wave Rectifier

### Operation

- Uses two diodes with center-tap transformer.

- Conducts during both half-cycles of AC.

- Output has double frequency pulses compared to half wave.

### Circuit Diagram

AC Source (center-tapped) → Two Diodes → Load Resistor (R_L)

### Key Points

- Positive output for both half-cycles.

- More efficient than half wave.

### Efficiency

[
\eta \approx 81.2%
]

## 4. Bridge Rectifier

### Operation

- Uses four diodes in a bridge configuration.

- No center-tap required.

- Conducts both half-cycles, providing full-wave rectification.

### Circuit Diagram

~

AC ---|>|---+

|     |

+---R_L---+

|     |

AC ---|<|---+

### Advantages

- Does not require center-tap transformer

- Higher output voltage than full-wave with center tap

## 5. Harmonic Components in Rectifier Circuits

- Output DC from rectifiers is pulsating → contains AC ripples.

- Fourier analysis shows fundamental frequency + harmonics:

  - Half-wave: 1st, 2nd, 3rd harmonics

  - Full-wave: 2nd, 4th, 6th harmonics

Implication:

- Filters are used to remove AC components and get smooth DC.

## 6. Filters

Filters reduce ripple in rectifier output. Common types:

### 6.1 Inductor (L) Filter

- Inductor connected in series with load.

- Smooths current by opposing sudden changes.

Effect:

- Reduces ripple current

- Voltage drop across inductor depends on load

Diagram:

Rectifier → L → Load

### 6.2 Capacitor (C) Filter

- Capacitor connected parallel to load.

- Charges during peak voltage, discharges during low voltage.

Effect:

- Provides steady voltage across load

- More effective for full-wave rectifier

Diagram:

Rectifier → Load

|

C

### 6.3 L-section Filter

- Combination of series inductor + parallel capacitor.

Effect:

- More efficient than single L or C filter

- Reduces ripple further

Diagram:

Rectifier → L → Load → C → Ground

### 6.4 π-section Filter

- Combination of C-L-C (capacitor-inductor-capacitor).

Effect:

- Provides very smooth DC

- Used in regulated power supplies

Diagram:

Rectifier → C → L → C → Load

## Summary Table – Rectifiers and Filters

| Topic | Description | Key Points |
| --- | --- | --- |
| Half Wave Rectifier | One diode, conducts half-cycle | Efficiency ~ 40.6% |
| Full Wave Rectifier | Two diodes, center-tap | Efficiency ~ 81.2% |
| Bridge Rectifier | Four diodes, no center-tap | Full-wave output, higher voltage |
| Harmonics | AC components in DC | Use filters to reduce |
| Inductor Filter | Series L | Reduces ripple current |
| Capacitor Filter | Parallel C | Smooth voltage |
| L-section Filter | L + C | Better ripple reduction |
| π-section Filter | C + L + C | Very smooth DC, used in regulated supply |

Key Points

- Diodes convert AC → DC (rectification).

- Half-wave rectifiers use one diode, bridge rectifiers use four diodes.

- Ripple is reduced using filters: L, C, L-section, π-section.

- Bridge rectifiers are widely used because they do not require a center-tap transformer.

# UNIT III – BIPOLAR JUNCTION TRANSISTOR (BJT)

A Bipolar Junction Transistor (BJT) is a three-terminal semiconductor device used for amplification and switching. It consists of two PN junctions forming either NPN or PNP types.

## 1. Principle of Operation

- BJT has three terminals:

  - Emitter (E) – Emits charge carriers

  - Base (B) – Thin, lightly doped region

  - Collector (C) – Collects carriers

- NPN transistor operation:

  - Forward bias the base-emitter junction

  - Reverse bias the base-collector junction

  - Electrons flow from emitter → base → collector, providing amplification.

- PNP transistor operation:

  - Polarity reversed

  - Holes are the majority carriers

## 2. Transistor Current Components

- Emitter Current (I_E) – Total current leaving emitter
[
I_E = I_B + I_C
]

- Base Current (I_B) – Small current entering base

- Collector Current (I_C) – Main current flowing through collector
[
I_C \approx \beta I_B
]

- Current Gain:

  - DC Current Gain ((\beta)): (\beta = I_C / I_B)

  - Common Base Current Gain ((\alpha)): (\alpha = I_C / I_E) (~0.95–0.99)

Relationship:
[
\beta = \frac{\alpha}{1 - \alpha}, \quad \alpha = \frac{\beta}{\beta + 1}
]

## 3. Transistor as an Amplifier

- Transistors amplify small input signals.

- Small base current variations produce large collector current variations, which can drive a load.

Amplifier Configurations:

- Common Emitter (CE) – Most popular

- Common Base (CB)

- Common Collector (CC) – Also called emitter follower

## 4. Common Emitter (CE) Configuration

### Characteristics

- Input: Base-Emitter voltage ((V_{BE}))

- Output: Collector-Emitter voltage ((V_{CE}))

- Phase difference: 180° between input and output

- Current gain ((\beta)) is high

### Circuit Diagram

Vcc

|

Rc

|

C -----> |

|

Q (NPN)

B -----> |

Re

|

GND

Key Points

- Good voltage, current, and power gain

- Widely used for amplification

## 5. Common Base (CB) Configuration

### Characteristics

- Input: Emitter-Base voltage ((V_{EB}))

- Output: Collector-Emitter voltage ((V_{CE}))

- Phase difference: 0° (in-phase)

- Current gain ((\alpha)) < 1

Circuit Diagram

C -----> Rc ---- Vcc

|

Q (NPN)

B -----> Base

E -----> Input

Key Points

- High voltage gain, low current gain

- Used in high-frequency applications

## 6. Common Collector (CC) / Emitter Follower

### Characteristics

- Input: Base-Emitter voltage ((V_{BE}))

- Output: Emitter voltage ((V_E))

- Phase difference: 0° (in-phase)

- Voltage gain: ~1 (unity)

- Current gain ((\beta + 1)) is high

Circuit Diagram

Vcc ---- Rc

|

Q (NPN)

B -----> Input

E -----> Output

Key Points

- Excellent current gain, but voltage gain ~1

- Used as buffer to drive low-impedance loads

## 7. Summary Table – BJT Configurations

| Configuration | Input | Output | Phase Difference | Voltage Gain | Current Gain | Applications |
| --- | --- | --- | --- | --- | --- | --- |
| CE (Common Emitter) | V_BE | V_CE | 180° | High | High | General amplification |
| CB (Common Base) | V_EB | V_CE | 0° | High | <1 | High-frequency circuits |
| CC (Common Collector) | V_B | V_E | 0° | ~1 | High | Buffer / Impedance matching |

Key Points

- BJT is current-controlled device: small base current controls large collector current.

- CE configuration is most commonly used for amplifiers.

- CB configuration is used in high-frequency applications.

- CC configuration is used as a buffer because of high current gain.

# UNIT IV – TRANSISTOR BIASING AND STABILIZATION

Transistor biasing is used to set the transistor’s operating point (Q-point) for proper amplification. Proper biasing ensures the transistor works in the active region and avoids distortion.

## 1. Operating Point (Q-point)

- Q-point is the DC collector current (I_C) and collector-emitter voltage (V_CE) for no input signal.

- Determines the region of transistor operation:

  - Cut-off – I_C ≈ 0, V_CE ≈ V_CC

  - Active region – Linear amplification

  - Saturation – V_CE small, transistor fully ON

Importance:

- Ensures linear amplification without distortion.

## 2. DC Load Line

- Graphical tool for transistor analysis.

- Equation:
[
V_{CE} + I_C R_C = V_{CC}
]

- Steps to draw DC load line:

  - At I_C = 0 → V_CE = V_CC (X-intercept)

  - At V_CE = 0 → I_C = V_CC / R_C (Y-intercept)

  - Connect the points → DC load line

Intersection with transistor characteristic → Q-point.

## 3. Biasing Methods

### 3.1 Fixed Bias

- Resistor R_B connected to base, no feedback from collector.

Circuit:

Vcc

|

R_B

|

B ----> Q

C ----> Rc ---- Vcc

E ----> GND

Advantages: Simple
Disadvantages: Poor stability, Q-point shifts with β variation

### 3.2 Collector-to-Base Bias

- Resistor R_B connected from collector to base → provides feedback.

Circuit:

Vcc

|

Rc

|

C ----+

|

R_B

|

B ----> Q

E ----> GND

Advantages: Better stability than fixed bias
Disadvantages: Still moderately sensitive to β

### 3.3 Voltage Divider Bias (Self-Bias / Emitter Feedback Bias)

- Uses two resistors R1 and R2 to provide stable base voltage.

- Emitter resistor R_E provides negative feedback → stabilizes I_C.

Circuit:

Vcc

|

R1

|

B ----> Q

|

R2

|

GND

E ----> Re ---- GND

C ---- Rc ---- Vcc

Advantages:

- Excellent stability

- Most widely used biasing method

## 4. Bias Stability

Bias stability ensures I_C does not change significantly with variations in:

- V_BE (Base-Emitter voltage)

- I_CO (Reverse saturation current)

- β (current gain)

### 4.1 Stabilization Against V_BE

- Emitter resistor R_E provides negative feedback

- If I_C ↑ → V_E ↑ → V_BE ↓ → I_B ↓ → I_C decreases → stabilizes

### 4.2 Stabilization Against β

- Voltage divider bias minimizes effect of β variations

### 4.3 Stabilization Against I_CO

- Series resistor and emitter resistor reduce sensitivity

## 5. Bias Compensation using Diodes and Transistors

- Diode or transistor used to counteract temperature variations in V_BE.

- Example: Diode in series with base → diode forward voltage drop changes with temperature, compensating V_BE drop of transistor.

Example Diagram:

Vcc

|

Rb

|

D ---> B

E ---> Re ---> GND

C ---> Rc ---> Vcc

- As temperature rises:

  - V_BE decreases

  - Diode voltage decreases similarly → I_C remains stable

## 6. Summary Table – Biasing Techniques

| Biasing Type | Circuit Feature | Stability | Notes |
| --- | --- | --- | --- |
| Fixed Bias | Base resistor only | Poor | Simple but Q-point shifts with β |
| Collector-to-Base Bias | R_B from collector to base | Moderate | Better than fixed bias |
| Voltage Divider / Self-Bias | R1-R2 divider + Re | Excellent | Widely used, stabilizes against β & V_BE |
| Bias Compensation | Diode or transistor in series | High | Compensates temperature variations |

Key Points

- Biasing sets Q-point in the active region.

- Voltage divider (self-bias) is the most stable method.

- Stability requires resistors and feedback to counter variations in β, I_CO, and V_BE.

- Bias compensation using diodes ensures stability over temperature changes.

# UNIT V – FIELD EFFECT TRANSISTORS (FET) AND MOSFETs

FETs are voltage-controlled devices used for amplification and switching. Unlike BJTs, they have high input impedance and low noise.

## 1. Field Effect Transistor (FET)

### 1.1 Construction

- FET has three terminals:

  - Gate (G) – Controls current flow

  - Source (S) – Supplies carriers

  - Drain (D) – Collects carriers

- Two main types:

  - Junction FET (JFET) – Gate forms PN junction

  - MOSFET – Gate insulated by oxide layer

- Channels:

  - N-channel FET → Current carried by electrons

  - P-channel FET → Current carried by holes

### 1.2 Principle of Operation

- Voltage at gate (V_GS) controls current through the channel (I_D).

- Gate current (I_G) is almost zero (reverse-biased).

Modes of Operation:

- Ohmic / Linear region: V_DS small, behaves like resistor

- Active / Saturation region: V_DS > V_GS - V_th, I_D becomes constant, ideal for amplification

### 1.3 Pinch-Off Voltage (V_P)

- Voltage at which channel is fully pinched off and current saturates.

- For N-channel JFET: V_GS < 0 → current decreases

[
I_D = I_{DSS} \left(1 - \frac{V_{GS}}{V_P}\right)^2
]

Where:

- I_DSS = Drain current at V_GS = 0

- V_P = Pinch-off voltage

### 1.4 Volt-Ampere Characteristics of JFET

- Output Characteristics (I_D vs V_DS)

  - Saturation region → constant I_D

  - Ohmic region → I_D increases linearly with V_DS

- Transfer Characteristics (I_D vs V_GS)

  - Shows variation of I_D with V_GS

Example:

- N-channel JFET: I_DSS = 10 mA, V_P = -4V → I_D = 0 at V_GS = -4V

### 1.5 Comparison of BJT and FET

| Feature | BJT | FET |
| --- | --- | --- |
| Type | Current-controlled | Voltage-controlled |
| Input Impedance | Low (~kΩ) | High (~MΩ) |
| Noise | Higher | Lower |
| Gain | High (current gain β) | Moderate |
| Switching Speed | Moderate | High |
| Biasing | Base current needed | Gate voltage controls |

## 2. Metal Oxide Semiconductor FET (MOSFET)

### 2.1 Types of MOSFET

- Depletion Type

  - Normally ON at V_GS = 0

  - Channel can be enhanced or depleted by gate voltage

- Enhancement Type

  - Normally OFF at V_GS = 0

  - Conducts only when V_GS exceeds threshold voltage V_th

### 2.2 Construction

- Gate insulated from channel by thin oxide layer (SiO2) → extremely high input resistance.

- Terminals: Gate (G), Source (S), Drain (D)

### 2.3 Working Operation

#### Depletion Type N-Channel

- V_GS = 0 → channel conducts

- Negative V_GS → channel narrows → I_D decreases

- Positive V_GS → channel widens → I_D increases

#### Enhancement Type N-Channel

- V_GS < V_th → no conduction

- V_GS > V_th → channel forms → I_D flows

### 2.4 V-I Characteristics of MOSFET

- Depletion-type N-channel

- I_D vs V_DS curves for different V_GS

- Linear region → I_D ∝ V_DS

- Saturation region → I_D ≈ constant

- Enhancement-type N-channel

- Transfer characteristic: I_D rises sharply when V_GS > V_th

- Linear and saturation regions similar to JFET

### 2.5 Comparison of JFET and MOSFET

| Feature | JFET | MOSFET |
| --- | --- | --- |
| Gate Control | PN junction | Insulated (oxide layer) |
| Input Current | Small (~μA) | Nearly zero |
| Input Resistance | High (~MΩ) | Very high (~GΩ) |
| Control Type | Depletion mode only | Depletion & Enhancement |
| Applications | Low-noise amplifier | Digital circuits, Switching |

## 3. Applications of FET/MOSFET

- Amplifiers (low-noise preamps)

- Switches in digital circuits

- Voltage-controlled resistors

- Oscillators

Key Points

- FET is voltage-controlled, BJT is current-controlled.

- JFET has normally ON channel, MOSFET has depletion/enhancement types.

- MOSFET is widely used in digital electronics because of high input impedance and fast switching.

- Pinch-off voltage defines the saturation region for constant current operation.