# UNIT I – DIGITAL COMPUTERS

## 1. Introduction

- A Digital Computer is an electronic device that can perform arithmetic and logical operations automatically.

- It processes digital data, i.e., data in binary form (0s and 1s).

- Computers are classified based on size, functionality, and purpose:

  - Microcomputers

  - Minicomputers

  - Mainframe computers

  - Supercomputers

Applications:

- Education, Business, Engineering, Medical, Communication

## 2. Block Diagram of a Digital Computer

A digital computer consists of the following major components:

+----------------+

| Input Devices  |

+--------+-------+

|

v

+--------+-------+

| Central        |

| Processing     |

| Unit (CPU)     |

+--------+-------+

|                |

|                |

+-----v-----+    +-----v-----+

| Memory    |    | Output    |

| (RAM/ROM) |    | Devices   |

+-----------+    +-----------+

Explanation:

- Input Unit: Accepts data and instructions from the user (keyboard, mouse, scanner).

- Central Processing Unit (CPU): Brain of the computer. Performs operations.

  - ALU (Arithmetic Logic Unit): Performs arithmetic & logical operations.

  - Control Unit (CU): Directs data flow and controls other units.

- Memory Unit: Stores data and instructions temporarily (RAM) or permanently (ROM).

- Output Unit: Sends processed data to the user (monitor, printer).

## 3. Computer Organization, Design, and Architecture

- Computer Organization: How different components are connected and interact.

- Computer Design: Internal arrangement of hardware and micro-operations to achieve functionality.

- Computer Architecture: Overall structure and operational concepts of the computer, including instruction set and programming model.

Example:

- Two computers may have the same architecture, but one may have faster hardware → different organization.

## 4. Basic Computer Organization and Design

- Instruction Codes: Binary codes that the CPU understands to perform operations.

  - Example: ADD → 0001, SUB → 0010

- Computer Registers: Small, fast storage inside CPU to hold data, addresses, or instructions temporarily.

  - Examples: Accumulator (AC), Instruction Register (IR), Program Counter (PC)

- Computer Instructions: Set of commands the CPU can execute:

  - Memory Reference Instructions (MRI): Access memory (LOAD, STORE)

  - Register Reference Instructions: Operate on registers (CLEAR AC)

  - Input/Output Instructions: Interact with I/O devices (IN, OUT)

## 5. Timing and Control

- Timing: Sequence of operations controlled by the system clock.

- Control Signals: Direct movement of data and execution of instructions.

Instruction Cycle (Fetch-Decode-Execute):

- Fetch: Retrieve instruction from memory → IR

- Decode: CU decodes instruction

- Execute: Perform operation via ALU or I/O

Text Diagram of Instruction Cycle:

Start → Fetch → Decode → Execute → Store Result → Next Instruction

## 6. Memory Reference Instructions

- LOAD: Copy data from memory to register

- STORE: Copy data from register to memory

- ADD: Add memory content to register

- SUB: Subtract memory content from register

Example:

- AC = Accumulator, M = Memory location 10

  - LOAD 10 → AC = M[10]

  - ADD 11 → AC = AC + M[11]

  - STORE 12 → M[12] = AC

## 7. Input, Output, and Interrupt

- Input/Output: Mechanism to communicate with external devices

- Interrupt: Signal sent by device to CPU to request attention

Types of Interrupts:

- Hardware Interrupt → Generated by hardware devices (keyboard, mouse)

- Software Interrupt → Generated by programs (division by zero)

Example:

- Printer requests CPU to send next page → hardware interrupt

- Program calls OS function → software interrupt

## 8. Complete Computer Description (Text-Based)

- Input Unit: Keyboard → ASCII code

- Memory Unit: RAM, ROM → Stores instructions/data

- CPU:

  - Control Unit → Directs operations

  - ALU → Performs calculations

- Registers: AC, IR, PC

- Output Unit: Monitor, Printer

- Timing & Control: Clock pulse synchronizes all operations

## 9. Practice Examples

- Explain the function of CPU registers and give examples.

- Describe fetch-decode-execute cycle with a real example.

- Differentiate Computer Architecture and Organization.

- List types of interrupts and explain one example of each.

- For memory reference instruction: AC=5, M[10]=7, execute ADD 10 and STORE 11.

# UNIT II – CENTRAL PROCESSING UNIT (CPU)

## 1. Processor Organization

- The CPU is the brain of the computer. It performs arithmetic, logic, control, and data movement operations.

- Main components of CPU:

  - ALU (Arithmetic Logic Unit): Performs arithmetic and logical operations.

  - Registers: Temporary storage for instructions and data.

  - Control Unit (CU): Directs CPU operations.

Text Diagram of CPU Organization:

+------------------+

| Control Unit (CU)|

+--------+---------+

|

v

+--------+---------+

| Arithmetic Logic |

| Unit (ALU)       |

+--------+---------+

|

+-------+--------+

| Registers      |

| AC, IR, PC,... |

+----------------+

Example:

- CPU fetches instruction ADD R1, R2 → ALU adds contents of registers R1 and R2 → result stored back in R1.

## 2. Register Organization

- Registers are high-speed memory units inside CPU for temporary storage.

Common Registers:

| Register | Purpose |
| --- | --- |
| AC (Accumulator) | Stores intermediate arithmetic/logic results |
| IR (Instruction Register) | Holds current instruction |
| PC (Program Counter) | Points to next instruction in memory |
| MAR (Memory Address Register) | Holds memory address to read/write |
| MDR (Memory Data Register) | Holds data to be written or just read from memory |
| Flags | Indicate conditions like zero, carry, overflow |

Example:

- Execute LOAD R1, 10

  - PC points to instruction in memory

  - Instruction fetched into IR

  - Address 10 loaded into MAR

  - Value from memory[10] moved to R1

## 3. Instruction Cycle

- CPU executes instructions using Instruction Cycle (Fetch → Decode → Execute).

Steps:

- Fetch: Read instruction from memory → IR

- Decode: CU decodes instruction to identify operation

- Execute: ALU performs operation or moves data

- Store Result: Result written back to register or memory

Text Diagram of Instruction Cycle:

Memory → Fetch → Decode → Execute → Result Stored → Next Instruction

Example:

- Instruction: ADD R2, R3

  - Fetch: Read instruction

  - Decode: CU interprets "ADD"

  - Execute: ALU adds contents of R2 and R3

  - Store: Result in R2

## 4. Control Unit

- Control Unit (CU) manages data flow and instruction execution.

Types:

- Hardwired Control Unit: Uses fixed circuits for control signals.

  - Fast, less flexible

  - Example: Basic ALU operations in simple microprocessors

- Microprogrammed Control Unit: Uses microinstructions stored in control memory.

  - Flexible, easy to modify

Example:

- Hardwired CU: ADD instruction → fixed circuit executes operation

- Microprogrammed CU: Instruction decoded → sequence of microinstructions executed

## 5. 8086 Processor Architecture

- 8086 is a 16-bit microprocessor from Intel.

- Features:

  - 16-bit data bus and 20-bit address bus

  - Can address 1 MB memory

  - Supports minimum and maximum mode

Text Diagram of 8086 Organization:

+---------------------+       +--------------------+

| Bus Interface Unit   | <--> | Memory / I/O Bus   |

+---------------------+       +--------------------+

^

|

+---------------------+

| Execution Unit (EU) |

| ALU, Registers, Flags|

+---------------------+

## 6. 8086 Register Organization

- General Purpose Registers (GPRs): AX, BX, CX, DX → can be used for data, index, or pointer operations

- Segment Registers: CS, DS, SS, ES → define memory segments

- Pointer and Index Registers: SP, BP, SI, DI → stack and memory operations

- Flag Register: Indicates conditions (carry, zero, sign, overflow)

Example:

- AX = 16-bit accumulator → AH (high byte), AL (low byte)

## 7. Physical Memory Organization

- 8086 has 1 MB memory accessed via segment:offset addressing

- Physical address = (Segment × 16) + Offset

Example:

- Segment = 1000H, Offset = 0020H

- Physical address = 1000H × 16 + 0020H = 10020H

## 8. General Bus Operation

- CPU communicates with memory and I/O via address bus, data bus, control bus

- Data Bus: Transfers data

- Address Bus: Transfers addresses

- Control Bus: Control signals for read/write

Example:

- CPU wants memory[200H] → Address 200H placed on address bus → READ signal sent → data transferred via data bus

## 9. I/O Addressing Capability

- 8086 supports memory-mapped I/O and I/O-mapped I/O

- 16-bit I/O address → can address 64 KB I/O ports

## 10. Special Processor Activities

- Minimum Mode: Single processor system

  - Control signals generated internally

- Maximum Mode: Multiprocessor system

  - Control signals generated externally using bus controller (8288)

Example:

- Minimum mode → CPU directly generates read/write signals

- Maximum mode → CPU works with coprocessor for control

## 11. Timings

- Instruction execution depends on clock cycles

- 8086 timings:

  - T1: Address placed on bus

  - T2: Memory read/write

  - T3: Data transfer

  - Execution times depend on instruction type and addressing mode

Example:

- MOV AX, BX → Fetch instruction → Read BX → Transfer to AX

## 12. Practice Problems

- Explain the difference between hardwired and microprogrammed control units.

- Calculate physical address for Segment = 2000H, Offset = 0040H in 8086.

- List general purpose registers of 8086 and their functions.

- Explain instruction cycle with an example.

- Differentiate minimum and maximum mode operation of 8086.

# UNIT III – COMPUTER ARITHMETIC

## 1. Introduction

- Computer Arithmetic deals with representation and manipulation of numbers inside a computer.

- The CPU’s Arithmetic Logic Unit (ALU) performs all arithmetic operations such as addition, subtraction, multiplication, division, and logical operations.

Applications:

- Calculators, scientific computations, graphics, embedded systems, control systems

## 2. Arithmetic Logic Unit (ALU)

- ALU is a part of CPU that performs arithmetic operations (ADD, SUB, MUL, DIV) and logic operations (AND, OR, NOT, XOR).

- It receives operands from registers, performs operations, and stores results back in registers or memory.

Text Diagram of ALU:

+-----------------+

| Operand A       |

+-----------------+

|

v

+-----------------+

|   ALU           |

| + - * / & | ^ ~ |

+-----------------+

|

v

+-----------------+

| Result          |

+-----------------+

Example:

- ADD R1, R2 → ALU adds contents of R1 and R2 → stores result in R1

## 3. Integer Representation

- Computers store integers in binary format.

- Types of representation:

  - Unsigned Binary: Only positive numbers (0 to 2ⁿ - 1)

  - Signed Binary: Includes negative numbers

    - Sign-Magnitude: MSB = sign (0=positive, 1=negative)

    - Two’s Complement: Most common method for arithmetic

    - One’s Complement: Invert all bits for negative numbers

Example:

- +7 in 8-bit two’s complement → 00000111

- -7 in 8-bit two’s complement → 11111001

## 4. Integer Arithmetic

- Addition: Binary addition with carry

- Subtraction: Use two’s complement for negative numbers

- Overflow: When result exceeds number of bits

Example:

- 8-bit: 11001010 + 00110111 = 11110001 → valid result

- 10011011 + 10101001 → overflow occurs

## 5. Floating Point Representation

- Floating-point numbers represent real numbers (decimal numbers) in scientific notation:
[
\text{Number} = Mantissa \times Base^{Exponent}
]

- IEEE 754 standard:

  - Single Precision (32-bit): 1 bit sign, 8 bits exponent, 23 bits mantissa

  - Double Precision (64-bit): 1 bit sign, 11 bits exponent, 52 bits mantissa

Example:

- 5.25 in binary: 101.01 → Normalized: 1.0101 × 2²

## 6. Floating Point Arithmetic

- Operations performed like integers but exponents must match before addition/subtraction.

- Multiplication: Multiply mantissas, add exponents

- Division: Divide mantissas, subtract exponents

Example:

- Add 1.25 × 2³ and 1.5 × 2²

  - Convert to same exponent: 1.25 × 2³ + 0.75 × 2³ = 2 × 2³ = 1 × 2⁴

## 7. Multiplication Algorithms

- Binary Multiplication: Shift and add method

- Booth’s Algorithm: Efficient for signed numbers

- Array Multiplier: Uses parallel adders for speed

Example:

- 6 × 3 in binary:

  - 6 → 0110

  - 3 → 0011

  - Multiply → 000110 (decimal 18)

## 8. Division Algorithms

- Restoring Division: Subtract divisor, check remainder, restore if negative

- Non-Restoring Division: More efficient, avoids restoring step

- Binary Division: Same as decimal long division

Example:

- 15 ÷ 3 → 1111 ÷ 0011 = 0101 (quotient 5)

## 9. Data Transfer and Manipulation Instructions

- MOV: Transfer data between registers/memory

- ADD, SUB, MUL, DIV: Arithmetic instructions

- INC, DEC: Increment/decrement

- AND, OR, XOR, NOT: Logical operations

Example:

- MOV AX, 05H → AX = 5

- ADD AX, 03H → AX = 8

## 10. 8086 Assembly Language Basic Programs

Program 1: Add Two Numbers

MOV AX, 5       ; Load 5 into AX

MOV BX, 3       ; Load 3 into BX

ADD AX, BX      ; AX = AX + BX

Program 2: Subtract Two Numbers

MOV AX, 10      ; Load 10 into AX

MOV BX, 6       ; Load 6 into BX

SUB AX, BX      ; AX = AX - BX

Program 3: Multiply Two Numbers

MOV AX, 4

MOV BX, 3

MUL BX          ; AX = AX * BX

## 11. Practice Problems

- Represent -12 in 8-bit two’s complement.

- Add 0.75 × 2² and 0.5 × 2³ in floating-point.

- Multiply 5 × 3 using binary multiplication.

- Divide 12 ÷ 3 using restoring division algorithm.

- Write 8086 assembly code to subtract two numbers.

Sure! Here’s a detailed, copyable, theory-rich explanation for UNIT IV – Input-Output Organization and Memory Organization, similar to previous units.

# UNIT IV – INPUT-OUTPUT AND MEMORY ORGANIZATION

## 1. Peripheral Devices

- Peripheral devices are external devices connected to the CPU to input, output, or store data.

- Examples:

  - Input: Keyboard, Mouse, Scanner

  - Output: Monitor, Printer, Speaker

  - Storage: Hard Disk, SSD, CD-ROM

- Characteristics:

  - Speed of operation

  - Data transfer rate

  - Interface type (parallel/serial)

Example:

- Keyboard inputs characters to CPU via ASCII codes.

## 2. Input-Output Interface

- The I/O interface connects peripheral devices to the CPU.

- Functions:

  - Data buffering

  - Data conversion (serial ↔ parallel)

  - Control signaling

Text Diagram:

CPU <----> I/O Interface <----> Peripheral Device

- Registers in I/O interface:

  - Data Register (DR): Temporarily holds data

  - Control Register (CR): Control signals (read/write)

  - Status Register (SR): Shows device status

## 3. Asynchronous Data Transfer

- Data is transferred without a clock signal, using handshaking signals (Ready, Acknowledge).

- Example:

  - CPU reads from keyboard only when keyboard signals “ready”.

## 4. Modes of Transfer

- Programmed I/O (Polling): CPU checks status register continuously.

  - Simple but wastes CPU time.

- Interrupt-driven I/O: Peripheral sends interrupt signal to CPU when ready.

  - CPU executes other tasks until interrupt occurs.

- Direct Memory Access (DMA):

  - Transfers data directly between memory and device without CPU intervention.

  - Improves speed for large data transfers.

Example:

- Transferring a large image from disk to memory using DMA rather than CPU polling.

## 5. Priority Interrupt

- When multiple devices request interrupts, priority determines servicing order.

- Types:

  - Fixed priority: Predefined order of importance

  - Rotating priority: Priority rotates after each service

Example:

- Printer (low priority), Keyboard (high priority) → Keyboard interrupt served first

## 6. Input-Output Processor (IOP)

- IOP is a dedicated processor for handling I/O operations independently of CPU.

- Reduces CPU load.

- Intel 8089 IOP:

  - Works with 8086 CPU

  - Handles data transfer, arithmetic, and control for peripherals

  - Can operate in parallel with CPU

## 7. Memory Organization

- Memory is organized to store instructions and data efficiently.

### 7.1 Memory Hierarchy

| Level | Speed | Capacity | Cost | Example |
| --- | --- | --- | --- | --- |
| Registers | Fastest | Very Low | High | CPU Registers |
| Cache | Fast | Low | Expensive | L1/L2 Cache |
| Main Memory (RAM) | Moderate | Medium | Moderate | DRAM |
| Secondary Memory | Slow | Large | Low | HDD, SSD |
| Tertiary Memory | Slowest | Very Large | Very Low | Tape, Optical |

### 7.2 Auxiliary Memory

- Non-volatile memory for long-term storage

- Examples: Hard disk, SSD, magnetic tape

### 7.3 Associative Memory (Content Addressable Memory, CAM)

- Memory accessed by content instead of address

- Used in cache memory and networking

Example:

- Searching for a value in CAM → returns location immediately

### 7.4 Cache Memory

- Small, fast memory between CPU and main memory

- Stores frequently used data/instructions

- Reduces CPU waiting time

Types of Cache:

- Direct Mapped Cache: Each block maps to one line

- Fully Associative Cache: Any block can go to any line

- Set Associative Cache: Compromise between the two

Example:

- Frequently accessed variables in a loop stored in L1 cache for faster access

### 7.5 Virtual Memory

- Concept: Use disk storage as an extension of RAM

- Allows execution of large programs than physical memory

- Implemented using paging and segmentation

Example:

- Running a 2 GB program on 1 GB RAM → OS swaps pages in/out from disk

### 7.6 Memory Management Hardware

- Provides efficient access and protection of memory

- Components:

  - Memory Address Register (MAR)

  - Memory Data Register (MDR)

  - Memory Management Unit (MMU): Maps logical addresses to physical addresses

  - Protection Registers: Prevent unauthorized access

Example:

- Logical address 0x2000 → MMU translates to physical address 0x12000

## 8. Practice Problems

- Explain differences between programmed I/O, interrupt-driven I/O, and DMA.

- What is associative memory? Give an example.

- Calculate physical address for page 5, offset 100 in paging virtual memory.

- Compare cache memory types: direct mapped, fully associative, set associative.

- Explain Intel 8089 IOP and its advantages over CPU-managed I/O.

Sure! Here’s a detailed, copyable, theory-rich explanation for UNIT V – Pipeline and Vector Processing & Multiprocessors, in the same style as the previous units.

# UNIT V – PIPELINE AND VECTOR PROCESSING & MULTIPROCESSORS

## 1. Parallel Processing

- Parallel processing involves simultaneous execution of multiple instructions using multiple processing units.

- Goal: Reduce execution time, increase throughput.

- Types:

  - Bit-level parallelism: Operates on multiple bits simultaneously

  - Instruction-level parallelism (ILP): Multiple instructions executed concurrently

  - Data-level parallelism: Same operation on multiple data items

Example:

- Performing matrix addition of two large matrices in parallel by dividing rows among multiple processors.

## 2. Pipelining

- Pipelining is a technique where multiple instruction stages overlap in execution, like an assembly line.

- Stages of a Pipeline:

  - Fetch – Retrieve instruction from memory

  - Decode – Decode the instruction

  - Execute – Perform operation

  - Memory access – Read/write memory if required

  - Write back – Store result

Example:

- While instruction 1 is in execute stage, instruction 2 can be in decode stage, and instruction 3 in fetch stage.

### 2.1 Arithmetic Pipeline

- Pipeline designed specifically for arithmetic operations like addition, multiplication, division.

- Example: Multiplying two 32-bit numbers using a 4-stage arithmetic pipeline:

  - Split numbers into 8-bit blocks

  - Multiply blocks in parallel

  - Add intermediate results

  - Generate final product

### 2.2 Instruction Pipeline

- Used in CPU instruction execution.

- Objective: Increase CPU throughput (instructions per cycle).

- Hazards in Pipeline:

  - Structural hazard: Hardware cannot support all concurrent operations

  - Data hazard: Instruction depends on result of previous instruction

  - Control hazard: Branch instruction changes flow

### 2.3 RISC Pipeline

- RISC (Reduced Instruction Set Computer) uses a simplified set of instructions for fast execution.

- Advantages of RISC:

  - Shorter instruction cycles

  - Easier to pipeline

- CISC (Complex Instruction Set Computer): Complex instructions, harder to pipeline

Comparison:

| Feature | CISC | RISC |
| --- | --- | --- |
| Instruction | Complex, multiple cycles | Simple, one cycle |
| Pipelining | Difficult | Easy |
| Hardware | More complex | Less complex |

## 3. Vector Processing

- Vector processors operate on vectors of data instead of single elements.

- Efficient for scientific and engineering computations.

- Array processors are similar: multiple processing elements working in parallel on arrays.

Example:

- Vector addition: Add 1000-element vectors using 4 processing units → 250 elements per unit

## 4. Multiprocessors

- Multiprocessor system has two or more processors sharing memory or I/O.

- Goal: Increase speed, reliability, and fault tolerance.

### 4.1 Characteristics of Multiprocessors

- Multiple CPUs: Share memory and peripherals

- Concurrent execution: Several instructions executed simultaneously

- Scalability: Can add more processors

Example:

- Dual-core processor in a laptop or a 32-core server

### 4.2 Interconnection Structures

- Ways to connect multiple processors:

  - Bus: Single shared communication line

  - Crossbar switch: Direct connection between any processor and memory

  - Multistage network: Combination of switches for faster access

### 4.3 Interprocessor Arbitration

- Arbitration: Resolving conflicts when multiple processors access shared resources.

- Methods:

  - Centralized: Single controller grants access

  - Distributed: Processors coordinate among themselves

### 4.4 Interprocessor Communication and Synchronization

- Communication: Processors exchange data through shared memory or message passing

- Synchronization: Ensures orderly execution and avoids race conditions

- Techniques:

  - Semaphores

  - Mutex (Mutual Exclusion)

  - Locks

Example:

- Two processors updating the same bank account must synchronize to avoid incorrect balance.

## 5. Summary

- Pipeline Processing: Increases instruction throughput using overlapping stages

- Vector and Array Processors: Perform parallel operations on data arrays

- RISC vs CISC: RISC easier to pipeline, CISC has complex instructions

- Multiprocessors: Multiple processors share memory, require arbitration and synchronization

## 6. Practice Problems

- Explain the difference between instruction and arithmetic pipelining.

- Compare RISC and CISC architectures with examples.

- Describe vector processing and give an example of its application.

- Explain interprocessor synchronization with a practical example.

- What are the advantages of multiprocessor systems over single processors?