# UNIT I – INTRODUCTION TO SEMICONDUCTOR PHYSICS

## 1. Introduction

- Semiconductors are materials whose electrical conductivity lies between conductors and insulators.

- Their conductivity can be controlled by temperature, light, or doping.

- Applications: Diodes, Transistors, Solar Cells, LEDs, Integrated Circuits.

Key properties:

- Intrinsic conductivity is low.

- Conductivity increases with temperature.

- Can be doped to increase electron or hole concentrations.

## 2. Formation of P-type and N-type Semiconductors

### 2.1 Intrinsic Semiconductors

- Pure silicon (Si) or germanium (Ge) without impurities.

- Charge carriers: equal electrons (n) and holes (p).

Conductivity formula:
[
\sigma = q(n\mu_n + p\mu_p)
]
where:

- q = electron charge

- μₙ, μₚ = mobility of electrons and holes

### 2.2 Extrinsic Semiconductors

N-type:

- Doped with pentavalent atoms (P, As).

- Extra electrons → majority carriers, holes → minority.

P-type:

- Doped with trivalent atoms (B, Ga).

- Holes → majority carriers, electrons → minority.

Example:

- Si + Phosphorus → N-type

- Si + Boron → P-type

ASCII Diagram of PN Junction Formation:

P-region           N-region

-----             -----

holes (major)     electrons (major)

electrons(minor)  holes(minor)

## 3. PN Junction Diode

- Formed by joining P-type and N-type semiconductors.

- Acts as a rectifier: allows current in one direction.

### 3.1 Forward Bias

- P connected to +ve, N connected to −ve.

- Barrier reduces, current flows.

### 3.2 Reverse Bias

- P connected to −ve, N connected to +ve.

- Barrier increases, only small leakage current flows.

## 4. Volt-Ampere Characteristics

Diode Equation:
[
I = I_S (e^{V/(nV_T)} - 1)
]

- I_S = reverse saturation current

- V_T = thermal voltage (≈26 mV at 300K)

- n = ideality factor (1–2)

Example 1 – Forward Current Calculation:

- I_S = 10⁻¹² A, V = 0.7 V, n = 1, V_T = 0.026 V

[
I = 10^{-12} (e^{0.7/0.026}-1) \approx 0.543 \text{ A}
]

ASCII Plot for V-I Characteristic:

I

|

|          /

|         /

|        /

|_______/________ V

## 5. Current Components in a Diode

- Drift Current – minority carriers move due to electric field.

- Diffusion Current – carriers move due to concentration gradient.

- Total Current: Sum of drift and diffusion currents.

## 6. Temperature Dependence

- Reverse saturation current I_S increases with temperature.

- Forward voltage drop V_f decreases slightly with temperature.

Example: If I_S doubles for every 10°C increase, at 50°C the reverse current increases significantly.

## 7. Static and Dynamic Resistances

7.1 Static Resistance
[
R_s = \frac{V}{I}
]

7.2 Dynamic Resistance
[
r_d = \frac{dV}{dI}
]

Example:

- At V = 0.7 V, I = 1 mA, dI/dV = 40 mA/V → r_d = 1/40 ≈ 25 Ω

## 8. Equivalent Circuit of a Diode

|----|>|----|

|           |

Rs           Rp

- Series resistance Rs → real diode resistance

- Parallel resistance Rp → leakage path

## 9. Load Line Analysis

- Intersection of diode characteristic with external circuit determines Q-point.

Example:

- V_s = 5 V, R = 1 kΩ → Load line: I = (5−V)/1000

I

|

|       /

|      /

|_____/________ V

## 10. Diode Capacitances

### 10.1 Diffusion Capacitance (C_d)

- Due to charge storage in forward bias.
[
C_d = \frac{dQ}{dV}
]

### 10.2 Transition Capacitance (C_t)

- Due to depletion layer acting as capacitor in reverse bias.
[
C_t = \frac{\epsilon A}{W}
]

- A = junction area, W = width of depletion layer, ε = permittivity

## 11. Stepwise Examples

- Forward Bias Current: I_S = 10⁻¹² A, V = 0.6 V → I ≈ 0.096 A

- Dynamic Resistance: Forward current I = 1 mA, dI/dV = 40 mA/V → r_d = 25 Ω

- Load Line Intersection: V_s = 5 V, R = 1 kΩ → Q-point determined graphically

## 12. Applications of PN Junction

- Rectifiers, Voltage Regulators, Signal Modulators, LEDs, Photodiodes, Solar Cells

## 13. Summary Table

| Parameter | Formula | Notes |
| --- | --- | --- |
| Diode Equation | I = I_S (e^{V/(nV_T)} − 1) | Forward current |
| Static Resistance | R_s = V/I | Ratio |
| Dynamic Resistance | r_d = dV/dI | Differential |
| Diffusion Capacitance | C_d = dQ/dV | Forward bias |
| Transition Capacitance | C_t = εA/W | Reverse bias |
| Load Line | V = V_s − I R | Q-point analysis |

Absolutely! Here’s a fully expanded, copyable, and detailed version of UNIT II – Special Purpose Devices, structured just like the previous unit, with theory, diagrams (copyable ASCII), examples, and applications. This can be pasted into Word/Google Docs and saved as PDF.

# UNIT II – SPECIAL PURPOSE DEVICES

## 1. Breakdown Mechanisms in Semiconductor Diodes

Semiconductor diodes, when reverse biased, can withstand only a certain reverse voltage. Beyond this, breakdown occurs, and the diode conducts heavily.

### Types of Breakdown

- Zener Breakdown

  - Occurs in heavily doped diodes at low reverse voltages (<5V).

  - Strong electric field causes tunneling of electrons from valence to conduction band.

  - Temperature coefficient is negative (voltage decreases with temperature).

- Avalanche Breakdown

  - Occurs in lightly doped diodes at high reverse voltages (>5V).

  - Free electrons gain enough energy to create electron-hole pairs via collision.

  - Temperature coefficient is positive (voltage increases with temperature).

Example:

- A diode with V_BR = 6 V, reverse biased at 6.5 V → avalanche occurs → current increases sharply.

## 2. Zener Diode

- A diode designed to operate in the breakdown region.

- Used as a voltage regulator.

### 2.1 Characteristics

I

|

|        / Zener breakdown region

|       /

|      /

|_____/________ V

- Forward region similar to normal diode.

- Reverse region shows sharp breakdown at V_Z.

### 2.2 Use as a Simple Regulator

- Connect Zener diode in parallel with load.

- Maintains constant voltage V_Z across the load.

Example Calculation:

- Supply voltage V_s = 12 V, series resistor R_s = 1 kΩ, V_Z = 5 V

- Load current I_L = 5 mA

[
I_S = \frac{V_s - V_Z}{R_s} = \frac{12-5}{1000} = 7 \text{ mA}
]
[
I_Z = I_S - I_L = 7-5 = 2 \text{ mA (through Zener)}
]

## 3. Tunnel Diode

- Highly doped P-N junction → thin depletion region.

- Exhibits negative resistance region due to tunneling.

### 3.1 Principle of Operation

- Electron tunnels through junction at small forward bias.

### 3.2 Energy Band Diagram

P-side valence band   N-side conduction band

_______  <--- Tunneling

|       |

- At small V_f, current increases → negative resistance → used in oscillators and high-speed switching.

Characteristic Curve:

I

|     /\

|    /  \

|___/    \_______ V

Negative resistance region

Example:

- Forward bias 0.1 V → tunneling current starts.

- Peak current I_p = 10 mA, valley current I_v = 4 mA → negative resistance region exists between I_p and I_v.

## 4. Varactor Diode

- Operates in reverse bias.

- Junction capacitance varies with reverse voltage → used in tunable filters, RF circuits, and voltage-controlled oscillators.

Capacitance Formula:
[
C_j = \frac{C_0}{(1 + V_R / V_0)^{1/2}}
]

- V_R = applied reverse voltage, V_0 = built-in potential, C_0 = zero-bias capacitance

Example:

- V_R = 5 V, C_0 = 50 pF, V_0 = 1 V → C_j ≈ 20 pF

## 5. Silicon Controlled Rectifier (SCR)

- Three-terminal device: Anode (A), Cathode (K), Gate (G).

- Operates as a controlled switch.

### 5.1 Principle of Operation

- SCR remains off until a gate pulse is applied.

- Once conducting, it stays on until current falls below holding current.

V-I Characteristics:

I

|        /

|       /

|      /   Gate trigger

|_____/________ V

Applications:

- AC/DC converters, motor control, light dimmers.

Example:

- Gate pulse applied → SCR starts conducting → load voltage appears.

## 6. Unijunction Transistor (UJT)

- Three terminals: Emitter (E), Base1 (B1), Base2 (B2)

- Exhibits negative resistance region.

### 6.1 Principle of Operation

- Emitter voltage > peak voltage → UJT conducts → current flows through emitter.

### 6.2 V-I Characteristics

I

|       /\

|      /  \

|_____/    \_______ V

- Used in oscillators, pulse generators, triggering circuits.

Example:

- Peak voltage V_P = 5 V, valley voltage V_V = 2 V → UJT triggers at V_P.

## 7. Summary Table of Devices

| Device | Operation | Key Feature | Applications |
| --- | --- | --- | --- |
| Zener Diode | Reverse bias breakdown | Voltage regulator | Power supply |
| Tunnel Diode | Forward tunneling | Negative resistance | Oscillators |
| Varactor Diode | Reverse bias | Variable capacitance | Tuned circuits |
| SCR | Controlled switch | Gate-triggered | AC/DC control |
| UJT | Emitter triggered | Negative resistance | Pulse generator |

## 8. Stepwise Examples

- Zener Regulator Current Calculation

- V_s = 12 V, R_s = 1 kΩ, V_Z = 6 V, I_L = 4 mA
[
I_s = \frac{12-6}{1000} = 6 \text{ mA}, \quad I_Z = 6-4 = 2 \text{ mA}
]

- Varactor Capacitance Calculation

- C_0 = 50 pF, V_R = 4 V, V_0 = 1 V
[
C_j = \frac{50}{(1+4/1)^{1/2}} = \frac{50}{\sqrt{5}} \approx 22.36 \text{ pF}
]

- SCR Triggering Example

- Peak load current 2 A, holding current 50 mA → SCR remains on until I < 50 mA

Absolutely! Here’s a fully expanded, copyable, detailed version of UNIT III – Rectifiers and Filters for your semiconductor/electronics subject. It includes theory, formulas, step-by-step examples, and diagrams (copyable ASCII style).

# UNIT III – RECTIFIERS AND FILTERS

## 1. Introduction

- Rectifiers are circuits that convert AC voltage to DC voltage.

- P-N junction diodes are used as rectifying devices because they allow current in one direction only.

- Filters smooth the pulsating DC from rectifiers to nearly pure DC.

## 2. P-N Junction Diode as a Rectifier

- Diode conducts in forward bias → current flows.

- Diode blocks in reverse bias → current almost zero.

- When connected in AC circuit:

- Forward half cycle: Diode conducts, positive voltage appears across load.

- Reverse half cycle: Diode blocks, load sees no voltage.

ASCII diagram:

AC Source

~

|

|---->|----+---- Load

|          |

~          |

-

## 3. Half-Wave Rectifier

- Uses one diode.

- Passes only one half of AC cycle.

### 3.1 Output Voltage

- Peak output voltage: ( V_m )

- Average (DC) voltage:
[
V_{DC} = \frac{V_m}{\pi}
]

- RMS voltage:
[
V_{RMS} = \frac{V_m}{2}
]

- Ripple factor:
[
r = \frac{V_{AC}}{V_{DC}} = 1.21
]

Example:

- V_m = 10 V → V_DC = 10/π ≈ 3.18 V

- V_RMS = 10/2 = 5 V

ASCII Output Waveform:

V

|     |     |

|     |     |

|     |     |

|_____|_____|_____ t

## 4. Full-Wave Rectifier

- Uses two diodes (center-tap transformer).

- Both halves of AC cycle are used.

### 4.1 Output Voltage

- Average (DC) voltage:
[
V_{DC} = \frac{2 V_m}{\pi}
]

- RMS voltage:
[
V_{RMS} = \frac{V_m}{\sqrt{2}}
]

- Ripple factor:
[
r = 0.482
]

ASCII Output Waveform:

V

|\/|\/|\/|

|  |  |  |

|  |  |  |

|__|__|__| t

Example:

- V_m = 10 V → V_DC = 6.37 V, V_RMS = 7.07 V

## 5. Bridge Rectifier

- Uses four diodes in bridge configuration.

- No center-tap transformer required.

- Both halves of AC pass through load in same direction.

ASCII Diagram of Bridge Rectifier:

+---->|----+

AC ~   |         |

|         |

+---->|----+

|         |

Load       AC ~

- Peak inverse voltage (PIV) for each diode = V_m (half-wave = 2 V_m)

- Average voltage: same as full-wave.

Example:

- V_m = 12 V → V_DC ≈ 7.64 V, Ripple factor r ≈ 0.482

## 6. Harmonic Components in Rectifier Circuits

- Rectified output contains DC component + AC harmonics.

- Fourier series analysis:

[
v(t) = V_{DC} + \sum_{n=1}^{\infty} V_n \sin(n\omega t + \phi_n)
]

- Half-wave rectifier → more harmonics → higher ripple

- Full-wave/bridge rectifier → fewer harmonics → smoother output

## 7. Filters (Qualitative Treatment)

- Filters reduce AC ripple in rectified output → smooth DC.

- Common types:

### 7.1 Capacitor Filter

- Connected parallel to load.

- Charges to peak voltage → discharges when diode stops conducting.

- Reduces ripple significantly.

### 7.2 Inductor (Choke) Filter

- Connected series with load.

- Smooths current due to inductor opposition to change in current.

### 7.3 LC Filter

- Series inductor + parallel capacitor.

- Most effective in reducing ripple.

### 7.4 CLC (π) Filter

- Capacitor – Inductor – Capacitor.

- Produces very smooth DC, widely used in regulated power supplies.

ASCII Diagram of Capacitor Filter:

Rectifier Output ----+---- Load

|

C

|

GND

## 8. Examples

- Half-Wave Rectifier DC Voltage

- V_m = 15 V
[
V_{DC} = \frac{V_m}{\pi} = 4.77 V
]
[
V_{RMS} = \frac{V_m}{2} = 7.5 V
]
Ripple factor: r = 1.21

- Full-Wave Rectifier DC Voltage

- V_m = 15 V
[
V_{DC} = \frac{2 V_m}{\pi} = 9.55 V
]
[
V_{RMS} = \frac{V_m}{\sqrt{2}} = 10.6 V
]
Ripple factor: r = 0.482

- Bridge Rectifier DC Voltage

- V_m = 12 V → V_DC ≈ 7.64 V, r ≈ 0.482

## 9. Summary Table

| Rectifier Type | Diodes | V_DC (avg) | Ripple Factor | Transformer |
| --- | --- | --- | --- | --- |
| Half-Wave | 1 | V_m/π | 1.21 | Optional |
| Full-Wave (CT) | 2 | 2V_m/π | 0.482 | Required |
| Bridge | 4 | 2V_m/π | 0.482 | Not needed |

| Filter Type | Connection | Purpose |
| --- | --- | --- |
| Capacitor | Parallel | Reduce ripple |
| Inductor | Series | Smooth current |
| LC | L-Series + C-Parallel | Very smooth DC |
| CLC (π) | C-L-C | Best ripple reduction |

# UNIT IV – BIPOLAR JUNCTION TRANSISTOR (BJT)

## 1. Introduction

- A BJT is a three-terminal semiconductor device that operates using both electrons and holes (hence “bipolar”) and can amplify current.

- It is used in amplifiers, switches, oscillators, and digital circuits.

Terminals:

- Emitter (E): Heavily doped, injects majority carriers into the base.

- Base (B): Thin and lightly doped; controls the number of carriers reaching the collector.

- Collector (C): Moderately doped; collects carriers from the emitter through the base.

BJT Types:

- NPN Transistor: Majority carriers are electrons; more commonly used due to higher mobility.

- PNP Transistor: Majority carriers are holes.

Applications:

- Signal amplification

- Switching in digital circuits

- Oscillator circuits

- Voltage regulation

## 2. BJT Construction and Working Principle

- NPN Transistor: Emitter injects electrons, base allows most electrons to reach the collector.

- PNP Transistor: Emitter injects holes, base allows holes to reach the collector.

- Forward active mode: Base-emitter junction is forward-biased, collector-base junction is reverse-biased.

- Saturation mode: Both junctions are forward-biased → transistor acts as a switch “ON.”

- Cutoff mode: Both junctions reverse-biased → transistor acts as a switch “OFF.”

Key Current Relations:
[
I_E = I_C + I_B
]
[
\beta = \frac{I_C}{I_B} \quad ; \quad \alpha = \frac{I_C}{I_E}
]
[
\beta = \frac{\alpha}{1-\alpha} \quad ; \quad \alpha = \frac{\beta}{\beta+1}
]

Example:

- β = 100, I_B = 30 μA → I_C = 3 mA, I_E = 3.03 mA

## 3. BJT as an Amplifier

- Small variations in base current produce large changes in collector current.

- The current gain is given by β.

- Voltage gain:
[
A_v = \frac{V_{out}}{V_{in}} = - \frac{R_C}{R_E} \quad (\text{for CE amplifier})
]

- Power gain: Product of voltage gain and current gain.

Example:

- I_B = 20 μA, β = 100, R_C = 2 kΩ, V_CC = 12 V → I_C = 2 mA, Voltage drop across R_C = I_C * R_C = 4 V

## 4. Configurations of BJT

### 4.1 Common Emitter (CE)

- Input: Base-Emitter

- Output: Collector-Emitter

- Characteristics: High voltage and current gain, 180° phase shift between input and output

- Applications: Audio/video amplifiers, general-purpose amplifiers

Example:

- V_CC = 12 V, R_C = 1 kΩ, I_B = 25 μA, β = 100 → I_C = 2.5 mA

- Collector voltage: V_C = V_CC - I_C*R_C = 12 - 2.5 = 9.5 V

### 4.2 Common Base (CB)

- Input: Emitter-Base

- Output: Collector-Base

- Characteristics: Low input resistance, high output resistance, current gain α < 1

- Applications: RF amplifiers, impedance matching at high frequencies

### 4.3 Common Collector (CC) / Emitter Follower

- Input: Base

- Output: Emitter

- Characteristics: Voltage gain ≈ 1, high current gain, low output resistance

- Applications: Impedance matching circuits, buffer stages

Example:

- Input voltage 5 V → output voltage ≈ 4.3 V (V_BE drop ≈ 0.7 V)

## 5. Transistor Operating Point (Q-Point)

- Determines collector current (I_C) and collector-emitter voltage (V_CE) with no input signal.

- DC load line equation:
[
I_C = \frac{V_{CC} - V_{CE}}{R_C}
]

- Biasing methods:

  - Fixed Bias – simplest, poor stability

  - Collector-to-Base Bias – moderate stability

  - Voltage Divider Bias – most stable

  - Emitter Bias – thermal stability improved

Example:

- V_CC = 15 V, R_C = 1 kΩ, R_E = 500 Ω, I_B = 20 μA, β = 100

- I_C = β*I_B = 2 mA

- V_CE = V_CC - I_C*(R_C + R_E) = 15 - 2*1.5 = 12 V

## 6. BJT Regions of Operation

| Region | V_BE | V_CE | Function |
| --- | --- | --- | --- |
| Cutoff | < 0.7 V | High | Switch OFF |
| Active | ≈ 0.7 V | > 0.2 V | Amplifier |
| Saturation | > 0.7 V | Low | Switch ON |
| Reverse Active | < 0.7 V | High | Rarely used |

## 7. Biasing Techniques (Expanded)

- Fixed Bias: Single resistor to base; simplest but poor thermal stability

- Collector-to-Base Bias: Base resistor connected to collector; feedback improves stability

- Voltage Divider Bias: Two resistors form a voltage divider; widely used

- Emitter Bias: Adds resistor in emitter path; improves thermal stability

Example – Voltage Divider Bias:

- V_CC = 12 V, R1 = 10 kΩ, R2 = 5 kΩ → V_B = 4 V

- R_E = 1 kΩ, V_BE = 0.7 V → I_E ≈ (V_B - V_BE)/R_E = (4 - 0.7)/1 ≈ 3.3 mA

## 8. Practical Applications of BJT

- Amplifiers – CE for voltage gain, CB for high-frequency applications

- Switches – Saturation = ON, Cutoff = OFF

- Oscillators – Generating AC signals

- Voltage Regulation – Using BJT with Zener diode

## 9. Stepwise Example Problems

Problem 1: Find I_C and V_CE

- V_CC = 15 V, R_C = 2 kΩ, R_E = 500 Ω, β = 100, I_B = 20 μA

- I_C = β*I_B = 2 mA

- I_E = I_C + I_B = 2.02 mA

- V_CE = V_CC - I_CR_C - I_ER_E = 15 - 22 - 2.020.5 ≈ 11 V

Problem 2: Calculate collector voltage in CE amplifier

- V_CC = 12 V, R_C = 1 kΩ, I_C = 2 mA

- V_C = V_CC - I_CR_C = 12 - 21 = 10 V

This version now includes:

- More examples and stepwise calculations

- Expanded applications and biasing explanations

# UNIT V – FIELD EFFECT TRANSISTOR (FET & MOSFET)

## 1. Introduction to FET

- A Field Effect Transistor (FET) is a voltage-controlled semiconductor device that uses only one type of charge carrier (electrons or holes).

- Key features:

  - High input impedance (~10^6 – 10^12 Ω)

  - Low noise

  - Low power consumption

- Applications:

  - Amplifiers with high input resistance

  - Analog switches

  - Digital circuits (MOSFETs)

  - Oscillators and voltage-controlled devices

## 2. Construction of FET

### 2.1 JFET (Junction FET)

- Terminals: Gate (G), Drain (D), Source (S)

- Types: N-channel (electron conduction) and P-channel (hole conduction)

- Operation: Reverse biasing the gate controls the channel conductivity.

Example Concept:

- N-channel JFET, with V_GS = 0 → maximum current (I_DSS)

- V_GS negative → channel narrows → I_D decreases

- At pinch-off, current remains almost constant regardless of V_DS

### 2.2 MOSFET (Metal Oxide Semiconductor FET)

- Terminals: Gate, Source, Drain

- Gate insulated from channel by oxide layer, giving extremely high input resistance.

- Types:

  - Enhancement-type MOSFET – Normally OFF, gate voltage turns it ON

  - Depletion-type MOSFET – Normally ON, gate voltage reduces current

## 3. Principle of Operation

- Current flows from Drain (D) to Source (S)

- Gate-Source voltage (V_GS) controls the channel width → controls Drain current (I_D)

### Key Terms:

- I_DSS: Maximum drain current at V_GS = 0 (for JFET)

- V_P (Pinch-Off voltage): Gate voltage at which the channel closes, and I_D saturates

JFET Equation (N-channel):
[
I_D = I_{DSS} \left(1 - \frac{V_{GS}}{V_P}\right)^2
]

Example Calculation:

- N-channel JFET: I_DSS = 12 mA, V_P = -3 V, V_GS = -1 V
[
I_D = 12 \left(1 - \frac{-1}{-3}\right)^2 = 12 (1 - 0.333)^2 = 12*(0.667)^2 \approx 5.34\text{ mA}
]

## 4. Volt-Ampere Characteristics of FET

### 4.1 Regions of Operation

- Ohmic/Linear Region:

  - V_DS small, FET behaves like a variable resistor

  - Used for voltage-controlled resistors

- Saturation/Active Region:

  - V_DS > V_DS(sat) → current I_D remains almost constant

  - Used for amplification

- Cutoff Region:

  - V_GS < V_P → channel closed → I_D ≈ 0

  - Device acts as a switch OFF

Example:

- V_P = -4 V, I_DSS = 10 mA, V_GS = -2 V → I_D = 2.5 mA (already calculated above)

## 5. Pinch-Off Voltage

- Definition: Voltage at which the conducting channel in a FET is fully depleted

- At pinch-off, the FET enters saturation region

- Determines maximum drain current control

Formula (for JFET):
[
I_D = I_{DSS} \left(1 - \frac{V_{GS}}{V_P}\right)^2
]

## 6. Comparison of BJT vs FET

| Feature | BJT | FET |
| --- | --- | --- |
| Type | Bipolar (electrons+holes) | Unipolar (electrons OR holes) |
| Input Impedance | Low (~kΩ) | High (~MΩ – GΩ) |
| Control | Current (I_B) | Voltage (V_GS) |
| Noise | Moderate | Low |
| Power Loss | Higher | Lower |
| Switching Speed | Medium | High |

## 7. MOSFET Overview

- Enhancement MOSFET: Normally OFF, gate voltage turns it ON

- Depletion MOSFET: Normally ON, gate voltage can turn it OFF

- MOSFET widely used in digital circuits (CMOS)

MOSFET Equation (Saturation Region):
[
I_D = K (V_{GS} - V_{TH})^2
]

- K = device constant

- V_TH = threshold voltage

Example:

- Enhancement N-MOSFET: V_TH = 2 V, V_GS = 5 V, K = 1 mA/V²
[
I_D = 1*(5-2)^2 = 9\text{ mA}
]

## 8. Applications of FET & MOSFET

- Voltage amplifiers – high input impedance → minimal loading on previous stage

- Analog switches – multiplexers, signal selection

- Digital circuits – CMOS technology for logic gates

- Power electronics – high-speed switching, power regulators

- Oscillators and signal generators – voltage-controlled devices

## 9. Summary

- FET = unipolar, voltage-controlled, used for low-noise, high-impedance applications

- BJT = bipolar, current-controlled, used for high-gain amplification

- MOSFET = insulated gate, high input impedance, widely used in digital and power applications

## 10. Practice Examples

Example 1: N-channel JFET with I_DSS = 8 mA, V_P = -5 V, find I_D for V_GS = -2 V
[
I_D = 8 \left(1 - \frac{-2}{-5}\right)^2 = 8 (0.6)^2 = 2.88\text{ mA}
]

Example 2: Enhancement N-MOSFET, V_TH = 2 V, V_GS = 4 V, K = 1.5 mA/V²
[
I_D = 1.5*(4-2)^2 = 6\text{ mA}
]

Example 3: Compare BJT and FET for switching:

- BJT requires base current I_B = 20 μA for I_C = 2 mA

- FET requires only V_GS = 2 V to achieve I_D = 2 mA

This now gives full, topic-wise, detailed coverage of UNIT V – FET & MOSFET including:

- Theory

- Formulas

- Regions of operation

- Examples

- Applications

- Comparison with BJT