# UNIT – I : BASIC STRUCTURE OF COMPUTERS

## 1. Introduction to Computer Systems

A computer is an electronic device that:

- Accepts data (input)

- Processes data using instructions (programs)

- Produces information (output)

- Stores data for future use

## 2. Computer Types

Computers are classified based on size, performance, and purpose.

### a) Supercomputers

- Extremely fast and powerful

- Used for weather forecasting, space research, simulations

- Example: Climate modeling systems

### b) Mainframe Computers

- Handle large amounts of data

- Support thousands of users

- Used in banks, airlines, government organizations

### c) Minicomputers

- Medium-sized computers

- Used in small organizations (mostly obsolete today)

### d) Microcomputers

- Personal computers

- Includes desktops, laptops, tablets, smartphones

### e) Embedded Systems

- Computers built into devices

- Used in washing machines, cars, medical devices

## 3. Functional Units of a Computer

A computer consists of several functional units:

### a) Input Unit

- Accepts data and instructions

- Examples: Keyboard, mouse, scanner

### b) Output Unit

- Displays processed results

- Examples: Monitor, printer, speakers

### c) Memory Unit

Stores data and instructions.

- Primary Memory: RAM, ROM

- Secondary Memory: Hard disk, SSD, pen drive

### d) Central Processing Unit (CPU)

The CPU is the brain of the computer.

#### Components of CPU

- ALU (Arithmetic Logic Unit)
Performs arithmetic and logical operations

- Control Unit (CU)
Directs all operations of the computer

- Registers
Small, fast memory inside CPU

## 4. Basic Operational Concepts

### Instruction Cycle

Every instruction goes through the following steps:

- Fetch instruction from memory

- Decode instruction

- Execute instruction

- Store result

This cycle is called the Fetch-Decode-Execute cycle.

## 5. Bus Structure

A bus is a communication pathway used to transfer data between components.

### Types of Buses

#### a) Data Bus

- Transfers actual data

- Bidirectional

#### b) Address Bus

- Transfers memory addresses

- Unidirectional

#### c) Control Bus

- Transfers control signals

- Coordinates operations

## 6. Structure of a Computer System

### Stored Program Concept

- Instructions and data are stored in memory

- CPU executes instructions sequentially

### Von Neumann Architecture

- Single memory for data and instructions

- Simple but slower due to bottleneck

### Harvard Architecture

- Separate memory for data and instructions

- Faster execution

## 7. Software

Software is a set of programs that instructs the computer.

### Types of Software

#### a) System Software

- Controls computer hardware

- Examples: Operating system, device drivers

#### b) Application Software

- Used by users

- Examples: MS Word, Excel, browsers

#### c) Utility Software

- Maintenance tools

- Examples: Antivirus, disk cleanup

## 8. Computer Performance

Performance refers to how fast and efficiently a computer works.

### Factors Affecting Performance

- CPU speed (clock rate)

- Number of cores

- Memory size and speed

- Cache memory

- I/O speed

### Performance Metrics

- Execution time

- Throughput

- Clock cycles per instruction (CPI)

## 9. Multiprocessors and Multicomputers

### Multiprocessors

- Multiple CPUs share memory

- Faster processing

- Example: Multi-core processors

### Multicomputers

- Independent computers connected via network

- Distributed memory

- Example: Computer clusters

## 10. Data Representation

Data inside computers is represented using binary system (0 and 1).

### Types of Data Representation

- Numbers

- Characters

- Instructions

## 11. Fixed Point Representation

### Definition

Fixed point representation stores numbers with a fixed number of digits after decimal point.

### Features

- Simple representation

- Limited range

- Less precision

### Example

Decimal: 12.75
Binary (fixed point): 1100.11

## 12. Floating-Point Representation

### Definition

Floating-point representation stores numbers using:

- Mantissa

- Exponent

### Standard

IEEE 754 standard

### Advantages

- Large range

- High precision

- Used in scientific calculations

### Example

( 1.25 \times 10^3 )

## 13. Comparison: Fixed vs Floating Point

| Feature | Fixed Point | Floating Point |
| --- | --- | --- |
| Range | Limited | Very large |
| Precision | Low | High |
| Complexity | Simple | Complex |
| Usage | Simple calculations | Scientific computing |

# REGISTER TRANSFER LANGUAGE AND MICRO OPERATIONS

## 1. Register Transfer Language (RTL)

### Definition

Register Transfer Language (RTL) is a symbolic notation used to describe the transfer of data between registers and the operations performed on the data inside a computer.

It shows:

- Which registers are involved

- What operation is performed

- When the operation occurs

### Example

R2 ← R1

This means: The contents of register R1 are transferred to register R2.

### Importance of RTL

- Helps in understanding internal CPU operations

- Used in processor design

- Simplifies explanation of micro-operations

## 2. Register Transfer Bus and Memory Transfers

### Common Bus System

A bus is a set of shared lines used to transfer data between registers.

- Reduces hardware complexity

- Allows multiple registers to share communication paths

### Register Transfer Using Bus

R1 → Bus → R2

### Memory Transfer Operations

#### Memory Read

DR ← M[AR]

(Data from memory location pointed by Address Register is copied to Data Register)

#### Memory Write

M[AR] ← DR

(Data from Data Register is stored in memory)

## 3. Micro Operations

### Definition

A micro operation is an elementary operation performed on data stored in registers.

### Types of Micro Operations

- Arithmetic

- Logic

- Shift

- Register Transfer

## 4. Arithmetic Micro Operations

Arithmetic micro operations perform basic arithmetic calculations.

### Examples

- Addition

- R3 ← R1 + R2

- Subtraction

- R3 ← R1 - R2

- Increment

- R1 ← R1 + 1

- Decrement

- R1 ← R1 - 1

### Hardware Used

- Binary adder

- Adder-subtractor circuits

## 5. Logic Micro Operations

Logic micro operations perform bitwise logical operations.

### Common Operations

- AND

- R3 ← R1 AND R2

- OR

- R3 ← R1 OR R2

- XOR

- R3 ← R1 XOR R2

- NOT

- R1 ← NOT R1

### Applications

- Masking bits

- Setting or clearing bits

- Comparing data

## 6. Shift Micro Operations

Shift operations move bits left or right within a register.

### Types of Shifts

#### Logical Shift

- Shifts bits and fills with zeros

- Used for unsigned numbers

#### Arithmetic Shift

- Preserves sign bit

- Used for signed numbers

#### Circular Shift

- Bits shifted out re-enter from opposite end

### Examples

R1 ← SHL R1   (Shift Left)

R1 ← SHR R1   (Shift Right)

## 7. Arithmetic Logic Shift Unit (ALSU)

### Definition

An Arithmetic Logic Shift Unit is a hardware unit that combines:

- Arithmetic operations

- Logic operations

- Shift operations

### Functions

- Performs all basic CPU computations

- Controlled by control signals

- Core part of the CPU

## 8. Instruction Codes

### Definition

An instruction code is a binary pattern that specifies:

- Operation to be performed

- Operand location

### Components of Instruction Code

- Opcode – Specifies the operation

- Operand – Specifies data or address

### Example

ADD R1, R2

## 9. Computer Registers

Registers are small, high-speed memory units inside the CPU.

### Common Registers

- Program Counter (PC) – Holds address of next instruction

- Instruction Register (IR) – Holds current instruction

- Memory Address Register (MAR)

- Memory Data Register (MDR)

- Accumulator (AC)

## 10. Computer Instructions

### Definition

Computer instructions are commands given to the CPU to perform operations.

### Types

- Data transfer instructions

- Arithmetic instructions

- Logical instructions

- Control instructions

## 11. Instruction Cycle

The Instruction Cycle is the sequence of steps CPU follows to execute an instruction.

### Phases

- Fetch

- Decode

- Execute

- Interrupt (if any)

### Fetch Cycle Example

IR ← M[PC]

PC ← PC + 1

## 12. Memory-Reference Instructions

These instructions access memory for operands.

### Examples

- LOAD

- STORE

- ADD

- SUBTRACT

### RTL Example

AC ← AC + M[AR]

## 13. Input–Output and Interrupt Instructions

### Input–Output Instructions

- Transfer data between CPU and I/O devices

- Examples: IN, OUT

### Interrupt

An interrupt is a signal that temporarily stops CPU execution to service an event.

### Types of Interrupts

- Hardware interrupt

- Software interrupt

- External interrupt

## 14. Instruction Formats

### Definition

Instruction format defines layout of bits in an instruction.

### Common Formats

- Zero-address instruction

- One-address instruction

- Two-address instruction

- Three-address instruction

### Example (One Address)

ADD X

## 15. Addressing Modes

### Definition

Addressing mode specifies how the address of an operand is obtained.

### Types of Addressing Modes

- Immediate

- Direct

- Indirect

- Register

- Register Indirect

- Indexed

- Relative

### Examples

- Immediate: MOV R1, #5

- Direct: LOAD A

- Register: ADD R1, R2

# UNIT – II: MICRO PROGRAMMED CONTROL AND MEMORY SYSTEM

## PART A: MICRO PROGRAMMED CONTROL

### 1. Control Unit (CU)

The Control Unit is a part of the CPU that controls all operations of the computer by:

- Generating control signals

- Coordinating data flow between CPU, memory, and I/O

- Directing ALU operations

### 2. Hardwired vs Microprogrammed Control

| Feature | Hardwired Control | Microprogrammed Control |
| --- | --- | --- |
| Implementation | Logic circuits | Control memory & microinstructions |
| Flexibility | Low | High |
| Speed | Fast | Slower |
| Modifications | Difficult | Easy |

### 3. Microprogrammed Control

#### Definition

Microprogrammed control uses microinstructions stored in control memory to generate control signals.

- Each machine-level instruction corresponds to a sequence of microinstructions.

### 4. Control Memory

- Special memory used to store microinstructions

- Each location contains control signals for micro-operations

- Readable like normal memory

#### Types of Microinstructions

- Horizontal: Each bit represents a control signal

- Vertical: Encoded representation of control signals (fewer bits)

### 5. Address Sequencing

Determines next microinstruction to execute:

- Sequential: Next microinstruction in memory

- Branching: Conditional jumps based on status

- Call & Return: Subroutine calls

#### Example

If Zero flag = 1 → Jump to microinstruction X

Else → Next microinstruction

### 6. Microprogram Examples

Example 1: Fetch Cycle

MAR ← PC

Read memory

IR ← M[MAR]

PC ← PC + 1

Example 2: Add Instruction

AC ← AC + R1

Microinstructions control data transfer between registers and ALU.

### 7. Design of Control Unit

#### Hardwired Control

- Uses combinational and sequential logic

- Generates control signals automatically

- Faster but rigid

#### Microprogrammed Control

- Uses control memory

- Microinstructions define signals

- Flexible and easy to modify

## PART B: MEMORY SYSTEM

### 1. Basic Concepts of Semiconductor RAM Memories

#### RAM (Random Access Memory)

- Volatile memory: loses data when power off

- CPU can read/write directly

#### Types of RAM

- Static RAM (SRAM): Fast, uses flip-flops, expensive

- Dynamic RAM (DRAM): Slower, uses capacitors, cheaper

### 2. Read-Only Memories (ROM)

#### ROM Types

- Masked ROM: Pre-programmed at manufacturing

- PROM: Programmable once

- EPROM: Erasable via UV light

- EEPROM: Electrically erasable

#### Use

- Stores firmware

- Boot programs

### 3. Cache Memories

#### Definition

Cache is small, fast memory placed between CPU and main memory to reduce access time.

#### Mapping Techniques

- Direct mapping

- Associative mapping

- Set-associative mapping

#### Write Policies

- Write-through: Immediately update memory

- Write-back: Update memory only when necessary

#### Performance Consideration

- Cache hit: Data found in cache → fast

- Cache miss: Data not found → fetch from main memory → slow

### 4. Virtual Memory

#### Definition

Virtual memory allows programs to use more memory than physically available by using secondary storage.

#### Techniques

- Paging: Divide memory into fixed-size blocks (pages)

- Segmentation: Divide memory into variable-size segments

#### Benefits

- Larger logical address space

- Efficient memory utilization

### 5. Secondary Storage

- Non-volatile storage used for permanent data storage

- Examples:

  - Hard disk drives (HDD)

  - Solid-state drives (SSD)

  - Optical disks (CD/DVD)

### 6. Introduction to RAID

RAID (Redundant Array of Independent Disks) combines multiple disks for:

- Reliability: Redundancy prevents data loss

- Performance: Parallel access increases speed

#### Common RAID Levels

- RAID 0: Striping → speed, no redundancy

- RAID 1: Mirroring → redundancy, no speed gain

- RAID 5: Striping + parity → balance between speed and reliability

## SUMMARY

- Control Unit controls data and operations

- Hardwired control is fast but rigid

- Microprogrammed control is flexible using control memory

- Registers and microinstructions perform operations in steps

- RAM and ROM store data temporarily and permanently

- Cache improves CPU performance

- Virtual memory enables larger logical memory

- Secondary storage and RAID ensure reliability and storage capacity

# UNIT – III: INPUT-OUTPUT ORGANIZATION

## PART A: PERIPHERAL DEVICES

### 1. Definition

Peripheral devices are external devices used to input, output, or store data in a computer.

### Types of Peripheral Devices

| Type | Examples | Function |
| --- | --- | --- |
| Input | Keyboard, Mouse, Scanner | Send data to CPU |
| Output | Monitor, Printer, Speaker | Display results |
| Storage | Hard Disk, SSD, USB | Store data permanently |
| Communication | Modem, Network Adapter | Transfer data between computers |

## 2. Input-Output Interface

The I/O Interface acts as a bridge between CPU and peripherals.

### Functions

- Transfers data between CPU and devices

- Converts data format if necessary

- Handles timing and control signals

### Components

- Control lines: Signals for operation (Read/Write)

- Data lines: Transfer actual data

- Status lines: Indicate device readiness

## PART B: DATA TRANSFER MODES

### 1. Asynchronous Data Transfer

- CPU and device operate at independent speeds

- Uses handshaking signals to synchronize

- Example: Reading data from a keyboard

### 2. Synchronous Data Transfer

- CPU and device operate at same clock speed

- No handshaking needed

## 3. Priority Interrupts

### Definition

Interrupts allow a peripheral to request CPU attention.

### Priority Scheme

- High-priority interrupts are serviced before low-priority

- Avoids CPU waiting for less critical tasks

### Interrupt Process

- Device sends interrupt signal

- CPU saves current state

- Executes Interrupt Service Routine (ISR)

- Returns to previous task

## 4. Direct Memory Access (DMA)

### Definition

DMA allows peripheral devices to transfer data directly to/from memory without CPU intervention.

### Advantages

- Faster data transfer

- Reduces CPU workload

### DMA Process

- Device requests DMA

- DMA controller transfers data to memory

- CPU resumes other operations

## 5. Input-Output Processor (IOP)

### Definition

An IOP is a specialized processor that handles all I/O operations.

### Functions

- Manages multiple I/O devices

- Reduces CPU involvement

- Controls data flow between peripherals and memory

## 6. Serial Communication

### Definition

Serial communication sends data one bit at a time over a single line.

### Advantages

- Requires fewer wires

- Suitable for long-distance communication

### Serial Protocols

- RS-232

  - Used for serial ports

  - Low-speed communication

- USB (Universal Serial Bus)

  - Hot-swappable, plug-and-play

  - Supports high-speed data transfer

- IEEE 1394 (FireWire)

  - High-speed serial interface

  - Used in multimedia applications

## PART C: PERIPHERAL COMPONENTS AND INTERCONNECTS

### 1. Peripheral Components

- Devices connected externally to the CPU

- Can include storage, input/output, and network devices

### 2. PCI (Peripheral Component Interconnect) Bus

- Standard local bus connecting CPU and peripherals

- High-speed communication between CPU and expansion cards

- Supports plug-and-play devices

- Transfers data in parallel mode

## SUMMARY

- Peripheral devices extend CPU capabilities for input, output, and storage

- I/O interfaces act as bridges between CPU and devices

- Asynchronous & synchronous modes manage data timing

- Interrupts allow CPU to respond to external events

- DMA and IOP reduce CPU workload for large data transfers

- Serial communication protocols like RS-232, USB, and IEEE 1394 standardize data transfer

- PCI bus connects peripherals to CPU efficiently

# UNIT – IV: OPERATING SYSTEMS OVERVIEW

## 1. Introduction to Operating Systems (OS)

### Definition

An Operating System (OS) is a software that acts as an interface between computer hardware and users, managing hardware resources and providing common services for applications.

### Functions of an Operating System

| Function | Description | Example |
| --- | --- | --- |
| Process Management | Manages execution of processes, scheduling, and multitasking | CPU allocation to programs |
| Memory Management | Allocates and deallocates memory to processes | Virtual memory management |
| File Management | Organizes, stores, retrieves, and protects files | File creation, deletion |
| Device Management | Manages input/output devices | Printer, keyboard, disk control |
| Security and Protection | Protects system and user data | User authentication, access control |
| Networking | Manages communication between computers | Internet, LAN |
| Error Detection | Monitors and handles hardware/software errors | Disk error detection, CPU faults |

## 2. Protection and Security

### Protection

- Mechanism to control access to resources

- Prevents unauthorized access

- Examples: Passwords, access permissions, firewalls

### Security

- Protects against malicious attacks and threats

- Ensures data integrity and confidentiality

- Examples: Anti-virus software, encryption

## 3. Distributed Systems

### Definition

A distributed system is a system where multiple computers work together to perform a task, appearing as a single system.

### Characteristics

- Multiple nodes connected via network

- Resource sharing

- Concurrent processes

- Fault tolerance

### Examples

- Cloud computing

- Online banking

- Distributed file systems

## 4. Special Purpose Systems

Special purpose OS are designed for specific tasks.

### Examples

| System | Purpose |
| --- | --- |
| Real-time OS | Industrial control, medical systems |
| Embedded OS | Washing machines, cars, mobile phones |
| Multimedia OS | Video/audio streaming and processing |

## 5. Operating System Structures

### 1. Monolithic Structure

- All OS functions are in a single large program

- Example: Early UNIX

### 2. Layered Structure

- OS divided into layers with well-defined functions

- Each layer uses services of lower layers

- Example: THE OS

### 3. Microkernel Structure

- Minimal core OS functions

- Other services run in user space

- Example: Mach OS, QNX

### 4. Modular Structure

- OS built from separable modules

- Easy to update and maintain

- Example: Modern Linux kernels

## 6. Operating System Services

| Service | Description |
| --- | --- |
| Program execution | Load and run programs |
| I/O operations | Manage device communication |
| File system manipulation | Create, delete, read, write files |
| Communication | Facilitate process communication |
| Error detection | Detect and correct errors |

## 7. System Calls

### Definition

System calls provide interface between application programs and OS.

### Types of System Calls

- Process Control: create, terminate processes

- File Management: open, read, write, delete files

- Device Management: request, release device

- Information Maintenance: get system date/time, process info

- Communication: send/receive messages

## 8. System Programs

System programs help users interact with the OS.

### Examples

- File management programs: copy, delete, list files

- Status information programs: monitor CPU, memory usage

- Development tools: compilers, editors

## 9. Operating System Generation

### Definition

Operating system generation involves building and configuring OS for specific hardware and requirements.

### Steps

- Load OS from storage to memory

- Customize OS with hardware-specific parameters

- Configure device drivers

- Start OS services

# CPU SCHEDULING AND DEADLOCK

## PART A: CPU SCHEDULING

### 1. Introduction

CPU scheduling determines which process will execute next when multiple processes are ready to run.

- Performed by the scheduler in the operating system

- Important for CPU utilization, response time, and system performance

### 2. Scheduling Criteria

| Criterion | Description |
| --- | --- |
| CPU Utilization | Keep CPU busy as much as possible (ideal 100%) |
| Throughput | Number of processes completed per unit time |
| Turnaround Time (TAT) | Total time taken from process submission to completion |
| Waiting Time (WT) | Time a process spends in ready queue |
| Response Time | Time from submission to first response (interactive systems) |
| Fairness | Each process gets a fair share of CPU |

### 3. Scheduling Algorithms

#### a) First-Come, First-Served (FCFS)

- Processes executed in order of arrival

- Non-preemptive

- Simple but may cause convoy effect (long processes block short ones)

#### b) Shortest Job Next (SJN) / Shortest Job First (SJF)

- Chooses process with shortest burst time

- Can be preemptive (SRTF) or non-preemptive

- Minimizes average waiting time

- Requires knowledge of CPU burst time

#### c) Priority Scheduling

- Processes assigned priority

- Higher priority executed first

- Can be preemptive or non-preemptive

- Problem: Starvation of low-priority processes

#### d) Round Robin (RR)

- Each process gets fixed time quantum

- Preemptive: CPU switches after quantum

- Fair and used in time-sharing systems

- Too small quantum → too many context switches

- Too large quantum → behaves like FCFS

#### e) Multilevel Queue Scheduling

- Processes divided into queues based on priority or type

- Each queue may have its own scheduling algorithm

- Example: Interactive vs batch processes

#### f) Multiple-Processor Scheduling

- Processes distributed among multiple CPUs

- Goals:

  - Load balancing

  - Efficient CPU utilization

- Methods:

  - Symmetric Multiprocessing (SMP): All CPUs share ready queue

  - Asymmetric Multiprocessing (AMP): One CPU schedules others

## 4. System Call Interface for Process Management

### Key System Calls

| System Call | Purpose |
| --- | --- |
| fork() | Creates a new process (child) |
| exit() | Terminates a process |
| wait() | Parent waits until any child terminates |
| waitpid() | Parent waits for a specific child to terminate |
| exec() | Replaces current process image with a new program |

## PART B: DEADLOCK

### 1. System Model

- Processes compete for resources (CPU, memory, I/O)

- Resources: Limited, can be allocated and released

- Deadlock occurs when processes are waiting indefinitely for resources held by others

### 2. Deadlock Characterization

Necessary Conditions (Coffman Conditions):

- Mutual Exclusion: Only one process can use resource at a time

- Hold and Wait: Process holding resources can request more

- No Preemption: Resources cannot be forcibly taken away

- Circular Wait: A closed chain of processes exists where each waits for the next

### 3. Deadlock Prevention

- Ensure at least one Coffman condition cannot hold:

- Mutual Exclusion: Use sharable resources if possible

- Hold and Wait: Require processes to request all resources at once

- No Preemption: Allow preemption of resources

- Circular Wait: Impose ordering of resource acquisition

### 4. Deadlock Avoidance

- Requires advance knowledge of resource requirements

- Example: Banker’s Algorithm

  - Resources allocated only if system remains in safe state

  - Avoids circular wait and unsafe allocation

### 5. Deadlock Detection

- Allows deadlock to occur, then detects it

- Methods:

  - Resource allocation graph: Detects cycles

  - Detection algorithms: Check for processes in waiting state

### 6. Recovery from Deadlock

- Process Termination:

  - Abort one or more processes to break deadlock

- Resource Preemption:

  - Temporarily take resources from processes

  - Rollback process state to safe point

# UNIT – V: MEMORY MANAGEMENT AND FILE SYSTEM

## PART A: MEMORY MANAGEMENT

### 1. Swapping

- Definition: Temporarily moves a process from main memory to secondary storage (swap space) and back.

- Purpose: Free memory for other processes.

- Example: Multiprogramming system swapping out inactive processes.

### 2. Contiguous Memory Allocation

- Definition: Each process occupies a single contiguous block of memory.

- Advantages: Simple and fast access.

- Disadvantages:

  - External fragmentation: Wasted memory between allocated blocks.

  - Difficult to grow process memory.

- Solution: Compaction (rearranging memory to remove fragmentation).

### 3. Paging

- Definition: Divides memory into fixed-size pages.

- Benefits: Eliminates external fragmentation.

- Page Table: Maps logical (virtual) addresses to physical addresses.

#### Page Table Structure

| Field | Description |
| --- | --- |
| Page Number | Index in logical memory |
| Frame Number | Index in physical memory |
| Valid/Invalid Bit | Indicates if page is in memory |

### 4. Segmentation

- Definition: Memory is divided into variable-sized segments based on logical divisions (code, data, stack).

- Advantages: Supports modular programming.

- Disadvantages: External fragmentation possible.

- Address: (Segment number, Offset)

### 5. Virtual Memory

- Definition: Allows execution of processes larger than physical memory by using secondary storage.

- Technique: Demand paging – pages loaded only when required.

### 6. Page-Replacement Algorithms

- Used when free frames are not available.

| Algorithm | Description |
| --- | --- |
| FIFO | Replaces the oldest page |
| LRU | Replaces the least recently used page |
| Optimal | Replaces page not needed for the longest time (ideal) |
| Clock | Efficient approximation of LRU |

### 7. Allocation of Frames

- Fixed allocation: Each process gets a fixed number of frames.

- Dynamic allocation: Frames allocated based on process size and priority.

### 8. Synchronization Hardware

- Ensures correct sequencing of processes accessing shared resources.

#### Example:

- Test-and-Set instruction

- Atomic hardware instructions to avoid race conditions

### 9. Semaphores

- Definition: Synchronization primitive to control access to resources.

- Types:

  - Binary semaphore: 0 or 1

  - Counting semaphore: Non-negative integers

#### Operations

- wait (P): Decrement semaphore

- signal (V): Increment semaphore

### 10. Classical Problems of Synchronization

- Producer-Consumer Problem: Synchronize producer and consumer processes.

- Dining Philosophers Problem: Avoid deadlock and starvation while accessing shared resources.

- Readers-Writers Problem: Synchronize reading and writing of shared data.

## PART B: FILE SYSTEM INTERFACE

### 1. Concept of a File

- Definition: A file is a collection of related information stored as a unit.

- Attributes: Name, type, location, size, access permissions

- Operations: Create, read, write, delete, rename

### 2. Access Methods

- Sequential Access: Read/write in order.

- Direct (Random) Access: Jump to a specific location.

- Indexed Access: Use index table for fast access.

### 3. Directory Structure

- Organizes files for easy access.

- Types:

  - Single-level

  - Two-level

  - Tree-structured

  - Acyclic graph

  - General graph

### 4. File System Mounting

- Attaching a file system to a directory of an existing file system

- Enables access to files across multiple storage devices.

### 5. File Sharing

- Multiple users or processes can access the same file.

- Requires access control and protection.

### 6. File Protection

- Mechanisms to prevent unauthorized access.

- Methods:

  - User ID / group ID

  - Access permissions (read, write, execute)

  - Access control lists (ACLs)

## PART C: FILE SYSTEM IMPLEMENTATION

### 1. File System Structure

- Layered design:

  - File system interface

  - File system implementation

  - Basic file system

  - I/O control

### 2. File System Implementation

- OS maintains metadata and control structures to manage files.

- Examples: inode table in UNIX, FAT table in DOS.

### 3. Directory Implementation

- Linear List: Simple but slow for large directories.

- Hash Table: Faster access, uses hashing on file names.

### 4. Allocation Methods

| Method | Description | Advantage | Disadvantage |
| --- | --- | --- | --- |
| Contiguous | Files occupy contiguous blocks | Fast access | External fragmentation |
| Linked | File blocks linked via pointers | No fragmentation | Sequential access slower |
| Indexed | Index block points to file blocks | Fast random access | Extra memory for index |

### 5. Free-Space Management

- Tracks unused blocks on disk.

- Methods:

  - Bit vector: 1 bit per block

  - Linked list: List of free blocks

  - Grouping / Counting: Reduce pointer overhead